# Aquifer Risk Map Methodology Description

# Introduction

The aquifer risk map is being developed to fulfill requirements of Senate Bill (SB 200). It is intended to help prioritize areas where domestic wells and state small water systems may be accessing groundwater that does not meet primary drinking water standards (maximum contaminant level or MCL). In accordance with SB 200, the risk map is to be made available to the public and is to be updated annually starting January 1, 2021. The Fund Expenditure Plan states the risk map will be used by Water Boards staff to help prioritize areas for available SAFER funding.

The aquifer risk map contains several data layers. The water quality layer compiles available de-clustered, depth-filtered water quality results, applies risk factors to those data, and ranks, by percentile, the relative risk of groundwater in an area not meeting primary drinking water standards. The domestic well density layer plots the density of domestic wells based on available well record data.  The state small water system layer shows the locations of state small water systems, based on data provided by Counties and other oversight agencies. Other reference layers that can be overlain on the map for reference include boundaries of priority areas in the Central Valley CV-SALTS program, Groundwater Sustainability Agency boundaries, and Disadvantaged Community data.

# Intended Use of this Analysis

The water quality risk ranking developed using this methodology are not intended to depict actual groundwater quality conditions at any given domestic supply well or small water system location. The purpose of this risk map analysis is to prioritize areas that may not meet primary drinking water standards for additional investigation and sampling efforts. The current lack of available domestic well and state small system water quality data makes it not possible to characterize the water quality for individual domestic wells and state small systems. The  analysis described herein represents a best effort at using the available data to estimate water quality risk for domestic wells and state small systems.

# Unit of Analysis

Groundwater quality risk is summarized by census block group. This allows the water quality risk to be combined with existing census information, such as disadvantaged community status and other demographic information. This also allows the data to be combined with the results of the Department of Water Resources Drought and Water Shortage Risk mapping, which identifies water accessibility risk throughout the state. Water quality data also can be viewed as individual well points and compiled into square mile public land survey sections. The well point and section-level data allow the user to better understand the potential distribution of available water quality data within a census block that contributed to the overall risk ranking for that block. State small system location data is available as point locations, and domestic well density information is available as both count per square mile and as count per census block group.

# Methodology

## Data Processing

Water quality results from the Division of Drinking Water (DDW), the US Geological Survey (USGS)-Groundwater Ambient Monitoring and Assessment (GAMA)programs’ Priority Basin and Domestic Well Projects, the USGS-National Water Information System dataset, the Department of Water Resources (DWR), local groundwater monitoring projects, and the Irrigated Lands Regulatory Program (AGLAND) were included in this analysis. Results were only included if the well met the depth-filtering criteria developed in Domestic Well Needs Assessment project. Data from all chemical constituents with a Maximum Contaminant Level (MCL) are assessed, and several additional chemical constituents including hexavalent chromium, copper, lead, and N-Nitrosodimethylamine (NDMA) are included in the analysis as well. Water quality results were converted to an MCL Index to allow comparison between chemical constituents (see Appendix A for table with MCLs). A more detailed presentation of data collection, data standardization, and data filtering all follow the steps outlined in the Needs Assessment Domestic Well Water Quality Tool White Paper.

## De-clustering

Available water quality results were spatially and temporally de-clustered to square mile sections to account for differences in data sampling density within each section over space and time. This was conducted to prevent certain areas with a high density of wells and frequent sampling to achieve a disproportionate weighting to the overall risk characterization of an area. To expand the coverage of the water quality risk map, averaged, de-clustered data from sections that contain a well(s) that provide  water quality data (“source sections”) are projected onto  neighboring sections that do not  include a well providing water quality data.

Water quality data is assessed using two metrics - the long-term (20 year) average and all recent results (within 2 years). The temporal and spatial de-clustering methodology for each metric is outlined below, and is further described in the Domestic Well Needs Assessment White Paper:

### Long-term average

- Water quality results from each well for each chemical constituent are averaged per year (for the past 20 years).

- The results from step one is averaged per well.

- The results from step two are averaged for all the wells that lie within a section.

- For sections that do not contain a well with water quality data, the de-clustered data from step three is projected onto adjacent sections.

### Recent results

- All recent (within the past 2 years) results in a section are categorized as “under” (less than 80 percent of MCL), “close” (80 percent – 100 percent of MCL), or “over” (greater than MCL).

- The count of recent results in each category are summarized per square mile section for each constituent.

- For square mile sections that do not contain a well with recent water quality data, the results from step two is averaged for all adjacent sections.

## Risk Factors

Water quality data for sections resulting from the analysis described above are summarized by census block group using a risk factor prioritization system. The prioritization is based on four water quality risk factors that are meant to capture different aspects of water quality risk based on the available data. Several additional informational fields included for reference.

| Risk Factor | Notation | Description |
| --- | --- | --- |
| Count of chemical constituents above MCL | CRF1 | How many individual chemical constituents have a long-term (20 year) average or recent result (within the past 2 years) above the MCL? |
| Count of chemical constituents within 80 percent of MCL | CRF2 | How many chemical constituents have a long-term average or recent result within 80 percent and 100 percent of the MCL? |
| Average MCL Index (for results above MCL) | CRF3 | For chemical constituents with a long-term average or recent result above the MCL, what is the average magnitude of those results? |
| Percent of high or medium risk sections | CRF4 | What percentage of square mile sections in the census block group are “high” or “medium” risk? High risk: contains at least one constituent with a long-term average or recent result above the MCL Medium risk: contains at least one constituent with a long-term average or recent result within 80 percent – 100 percent of the MCL |

| Reference Data | Description |
| --- | --- |
| List of chemical constituents above MCL | List of chemical constituents with a long-term or recent result above the MCL |
| List of chemical constituents within 80 percent of MCL | List of chemical constituents with a long-term or recent result within 80 percent – 100 percent of MCL. |
| Percent area with water quality data | The percentage of sections in the census block group that contain water quality data. |
| Estimated domestic wells | The total estimated count of domestic wells in the census block group based on data from DWR OSWCR. |
| Estimated domestic well population | The total estimated population served by domestic wells in the census block group based on data from the USGS road-enhanced methodology (Johnson and Belitz, 2019) |

These water quality risk factors are aggregated into a final water quality score, calculated as:

Water Quality Score=CRF1+CRF22+CRF310*(CRF4)

The water quality scores for all census block groups are converted into percentiles to normalize the scores. Higher scores and high percentiles indicate areas that are at relatively higher risk for water quality issues in domestic wells and state small water systems. Census block groups with a score of zero (no constituents above or close to the MCL) are automatically assigned to the 0th percentile and are not included in the percentile calculation. Approximately 33 percent of census blocks fall into this category. It is important to note that because of the data filtering and de-clustering involved in these calculations, a risk percentile of zero does not necessarily mean there is no water quality risk in an area (see discussion on areas with sparse/no available data, below).

In addition to the census block group percentiles, detailed data layers show water quality data summarized at the square mile section level and at the point (well) level. These layers display similar risk factors for sections and well points:

| Section Data Risk Information | Description |
| --- | --- |
| Section Risk Category | Is the section high, medium, or low risk? High: contains at least one constituent with a long-term average or recent result above the MCL Medium: contains at least one constituent with a long-term average or recent result within 80 percent – 100 percent of the MCL Low: contains no constituents with a long-term average or recent result greater than 80 percent of the MCL |
| Count of chemical constituents above MCL | How many chemical constituents in the section have a long-term (20 year) average or recent result (within the past 2 years) above the MCL? |
| Count of chemical constituents within 80 percent of MCL | How many chemical constituents in the section have a long-term (20 year) average or recent result (within the past 2 years) within 80 percent and 100 percent of the MCL? |
| Average MCL Index (for results above MCL) | For chemical constituents that are above the MCL in the section, what is the average magnitude of those results? (How high are the results above the MCL?) |
| List of chemical constituents above MCL | List of chemical constituents with a long-term or recent result above the MCL in the section |
| List of chemical constituents close to MCL | List of chemical constituents with a long-term or recent result within 80 percent – 100 percent of MCL in the section |

| Point Data Risk Information | Description |
| --- | --- |
| Well Risk Category | Is the well high, medium, or low risk? High: contains at least one constituent with a long-term average or recent result above the MCL Medium: contains at least one constituent with a long-term average or recent result within 80 percent – 100 percent of the MCL Low: contains no constituents with a long-term average or recent result greater than 80 percent of the MCL |
| Count of chemical constituents above MCL | How many chemical constituents in the well have a long-term (20 year) average or recent result (within the past 2 years) above the MCL? |
| Point Data Risk Information | Description |
| Count of chemical constituents within 80 percent of MCL | How many chemical constituents in the well have a long-term (20 year) average or recent result (within the past 2 years) within 80 percent and 100 percent of the MCL? |
| Average MCL Index (for results above MCL) | For chemical constituents that are above the MCL in the well, what is the average magnitude of those results? (How high are the results above the MCL?) |
| List of chemical constituents above MCL | List of chemical constituents with a long-term or recent result above the MCL in the well |
| List of chemical constituents close to MCL | List of chemical constituents with a long-term or recent result within 80 percent – 100 percent of MCL in the well |

## Areas with No Available Water Quality Data

Out of 23,212 census block groups, approximately 5,183 (22 percent) do not contain water quality data for any of the chemical constituents.

Most (3,250, or 63 percent) of these “no data” block groups are in the densely populated areas (i.e., Los Angeles and San Francisco Bay), where census block groups are very small (less than 1 square mile) and do not overlap a square mile section with a well with water quality data. These areas are predominantly served by municipal water systems and are less likely to have a significant population on domestic wells. However, there are several block groups with no data in Imperial County and parts of Humboldt, Trinity, and Shasta Counties, where there is likely populations of domestic well users. Currently, block groups with no water quality data are not included in the percentile ranking process and are assigned a water quality risk percentile of zero. Due to the data filtering and de-clustering involved in these calculations, a risk percentile score of “zero” does not mean there is not a water quality risk in each area.

## Areas with Sparse Available Water Quality Data

Additionally, there are 118 census block groups that contain sparse (less than 10 percent) data coverage. While these block groups are included in the percentile ranking, they are flagged on the tool with the hatch marks to alert the user.

## Individual Chemical Constituents

Single-chemical constituent layers are available as square mile section data for Nitrate, Arsenic, 1,2,3-Trichloropropane, Hexavalent Chromium, and Uranium. These layers display the long-term average and the count of recent results over, close to, and under the MCL per square mile section for a single chemical constituent.

# Appendix A – Chemical Abbreviations and Comparison Concentrations

| Chemical Abbreviation (Web Tool) | Chemical Name | Units | Comparison Concentration Value | Comparison Concentration Type |
| --- | --- | --- | --- | --- |
| 24D | 2,4-Dichlorophenoxyacetic acid (2,4 D) | UG/L | 70 | MCL |
| AL | Aluminum | UG/L | 1000 | MCL |
| ALACL | Alachlor | UG/L | 2 | MCL |
| ALPHA | Gross Alpha radioactivity | pCi/L | 15 | MCL |
| AS | Arsenic | UG/L | 10 | MCL |
| ATRAZINE | Atrazine | UG/L | 1 | MCL |
| BA | Barium | MG/L | 1 | MCL |
| BDCME | Bromodichloromethane (THM) | UG/L | 80 | MCL |
| BE | Beryllium | UG/L | 4 | MCL |
| BETA | Gross beta | pCi/L | 50 | MCL |
| BHCGAMMA | Lindane (Gamma-BHC) | UG/L | 0.2 | MCL |
| BIS2EHP | Di(2-ethylhexyl)phthalate (DEHP) | UG/L | 4 | MCL |
| BRO3 | Bromate | UG/L | 10 | MCL |
| BTZ | Bentazon | UG/L | 18 | MCL |
| BZ | Benzene | UG/L | 1 | MCL |
| BZAP | Benzo(a)pyrene | UG/L | 0.2 | MCL |
| BZME | Toluene | UG/L | 150 | MCL |
| CD | Cadmium | UG/L | 5 | MCL |
| CHLORDANE | Chlordane | UG/L | 0.1 | MCL |
| CHLORITE | Chlorite | MG/L | 1 | MCL |
| CLBZ | Chlorobenzene | UG/L | 70 | MCL |
| CN | Cyanide (CN) | UG/L | 150 | MCL |
| CR | Chromium | UG/L | 50 | MCL |
| CR6 | Chromium, Hexavalent (Cr6) | UG/L | 10 | Temporary comparison level* |
| CRBFN | Carbofuran | UG/L | 18 | MCL |
| CTCL | Carbon Tetrachloride | UG/L | 0.5 | MCL |
| CU | Copper | MG/L | 1.3 | Action Level |
| DALAPON | Dalapon | UG/L | 200 | MCL |
| DBCME | Dibromochloromethane (THM) | UG/L | 80 | MCL |
| DBCP | 1,2-Dibromo-3-chloropropane (DBCP) | UG/L | 0.2 | MCL |
| DCA11 | 1,1-Dichloroethane (1,1 DCA) | UG/L | 5 | MCL |
| DCA12 | 1,2 Dichloroethane (1,2 DCA) | UG/L | 0.5 | MCL |
| DCBZ12 | 1,2 Dichlorobenzene (1,2-DCB) | UG/L | 600 | MCL |
| DCBZ14 | 1,4-Dichlorobenzene (p-DCB) | UG/L | 5 | MCL |
| DCE11 | 1,1 Dichloroethylene (1,1 DCE) | UG/L | 6 | MCL |
| DCE12C | cis-1,2 Dichloroethylene | UG/L | 6 | MCL |
| DCE12T | trans-1,2, Dichloroethylene | UG/L | 10 | MCL |
| DCMA | Dichloromethane (Methylene Chloride) | UG/L | 5 | MCL |
| DCP13 | 1,3 Dichloropropene | UG/L | 0.5 | MCL |
| DCPA12 | 1,2 Dichloropropane (1,2 DCP) | UG/L | 5 | MCL |
| DINOSEB | Dinoseb | UG/L | 7 | MCL |
| DIQUAT | Diquat | UG/L | 20 | MCL |
| DOA | Di(2-ethylhexyl)adipate | MG/L | 0.4 | MCL |
| EBZ | Ethylbenzene | UG/L | 300 | MCL |
| EDB | 1,2 Dibromoethane (EDB) | UG/L | 0.05 | MCL |
| ENDOTHAL | Endothall | UG/L | 100 | MCL |
| ENDRIN | Endrin | UG/L | 2 | MCL |
| F | Fluoride | MG/L | 2 | MCL |
| FC11 | Trichlorofluoromethane (Freon 11) | UG/L | 150 | MCL |
| FC113 | 1,1,2-Trichloro-1,2,2-Trifluoroethane (Freon 113) | MG/L | 1.2 | MCL |
| GLYP | Glyphosate (Round-up) | UG/L | 700 | MCL |
| H-3 | Tritium | pCi/L | 20000 | MCL |
| HCCP | Hexachlorocyclopentadiene | UG/L | 50 | MCL |
| HCLBZ | Hexachlorobenzene (HCB) | UG/L | 1 | MCL |
| HEPTACHLOR | Heptachlor | UG/L | 0.01 | MCL |
| HEPT-EPOX | Heptachlor Epoxide | UG/L | 0.01 | MCL |
| HG | Mercury | UG/L | 2 | MCL |
| MOLINATE | Molinate | UG/L | 20 | MCL |
| MTBE | MTBE (Methyl-tert-butyl ether) | UG/L | 13 | MCL |
| MTXYCL | Methoxychlor | UG/L | 30 | MCL |
| NI | Nickel | UG/L | 100 | MCL |
| NNSM | N-Nitrosodimethylamine (NDMA) | UG/L | 0.01 | NL |
| NO2 | Nitrite as N | MG/L | 1 | MCL |
| NO3N | Nitrate as N | MG/L | 10 | MCL |
| OXAMYL | Oxamyl | UG/L | 50 | MCL |
| PB | Lead | UG/L | 15 | Action Level |
| PCA | 1,1,2,2 Tetrachloroethane (PCA) | UG/L | 1 | MCL |
| PCATE | Perchlorate | UG/L | 6 | MCL |
| PCB1016 | Polychlorinated Biphenyls (PCBs) | UG/L | 0.5 | MCL |
| PCE | Tetrachloroethene (PCE) | UG/L | 5 | MCL |
| PCP | Pentachlorophenol (PCP) | UG/L | 1 | MCL |
| PICLORAM | Picloram | MG/L | 0.5 | MCL |
| RA-226 | Radium 226 | pCi/L | 5 | MCL |
| RA-228 | Radium 228 | pCi/L | 5 | MCL |
| SB | Antimony | UG/L | 6 | MCL |
| SE | Selenium | UG/L | 50 | MCL |
| SILVEX | 2,4,5-TP (Silvex) | UG/L | 50 | MCL |
| SIMAZINE | Simazine | UG/L | 4 | MCL |
| SR-90 | Strontium 90 | pCi/L | 8 | MCL |
| STY | Styrene | UG/L | 100 | MCL |
| TBME | Bromoform (THM) | UG/L | 80 | MCL |
| TCA111 | 1,1,1-Trichloroethane | UG/L | 200 | MCL |
| TCA112 | 1,1,2-Trichloroethane | UG/L | 5 | MCL |
| TCB124 | 1,2,4- Trichlorobenzene (1,2,4 TCB) | UG/L | 5 | MCL |
| TCDD2378** | 2,3,7,8-Tetrachlorodibenzodioxin (Dioxin) | UG/L | 3.00E-05 | MCL |
| TCE | Trichloroethene (TCE) | UG/L | 5 | MCL |
| TCLME | Chloroform (THM) | UG/L | 80 | MCL |
| TCPR123 | 1,2,3-Trichloropropane (1,2,3 TCP) | UG/L | 0.005 | MCL |
| THIOBENCARB | Thiobencarb | UG/L | 70 | MCL |
| THM | Total Trihalomethanes | UG/L | 80 | MCL |
| TL | Thallium | UG/L | 2 | MCL |
| TOXAP | Toxaphene | UG/L | 3 | MCL |
| U | Uranium | pCi/L | 20 | MCL |
| VC | Vinyl Chloride | UG/L | 0.5 | MCL |
| XYLENES | Xylenes (total) | UG/L | 1750 | MCL |

*Since there is currently no MCL for Hexavalent Chromium (CR6), a temporary comparison value was to remain consistent with the risk assessment for public water systems.

**No data for 2,3,7,8-Tetrachlorodibenzodioxin (Dioxin) was available for this analysis, because there are no samples from wells that met our depth/time criteria.

The well record information is from the Department of Water Resources Online System for Well Completion Reports.

The small water system location data may not represent the actual location of the well head or of the service boundaries. Due to constraints in locating small water systems, the location may represent the administrative address, or another location associated with the system. Additionally, the locations for Monterey County were provided for all systems with 2 – 14 connections. To isolate the systems in Monterey County that fit the definition of state small water systems (5 – 14 connections) only systems that serve four or more APNs were included in this analysis (totaling 268 systems in Monterey County).

The comparison concentration values for chemicals without an MCL are as follows: Hexavalent Chromium – 10 micrograms per liter (µG/L); Copper – 1.3 milligrams per liter (MG/L); Lead – 15 µG/L; N-Nitrosodimethylamine (NDMA) – 0.1 µG/L.

See page 5 of the Domestic Well Needs Assessment White Paper. The MCL index consists of the finding divided by the MCL, with a special consideration for non-detect results with a reporting limit above the MCL.

To calculate the 20-year average, water quality results with sample collection dates between January 1, 2000 and January 1, 2020 were used.

To calculate results within the last two years, water quality results with sample collection dates between January 1, 2018 and January 1, 2020 were used.

For this map, on the “source” and “neighbor” sections described in the Domestic Well Needs Assessment White Paper are used. The White Paper also describes the calculations for “groundwater unit” sections, which are not included in this map.

To calculate the percentage of high and medium risk sections, the number of “high” sections plus the number of “medium” sections divided by two are divided by the total number of sections in the census block group. This is to assign more weight to “high” risk sections and less weight to “medium” risk sections.