# 2022 Aquifer Risk Map Methodology

This document describes the methodology for the 2022 Aquifer Risk Map. The methodology for the 2021 map can be found here.

# Introduction

The aquifer risk map is being developed to fulfill requirements of Senate Bill (SB 200, Monning, 2019), and is a component of California’s Safe and Affordable Fund for Equity and Resilience (SAFER) program. The aquifer risk map is intended to help prioritize areas where domestic wells and state small water systems may be accessing groundwater that does not meet primary drinking water standards (maximum contaminant level or MCL). In accordance with SB 200, the risk map is to be made available to the public and is to be updated annually starting January 1, 2021. SB 200 also requires that a Fund Expenditure Plan be developed annually. The Fund Expenditure Plan states the risk map will be used by Water Boards staff to help prioritize areas for available SAFER funding.

The aquifer risk map contains several data layers. The water quality risk layer compiles available de-clustered, depth-filtered water quality results, and identifies areas where either the long-term average or recent results are above the MCL (“high risk”). The domestic well density layer shows the density of domestic wells based on available well record data.  The state small water system layer shows the locations of state small water systems, based on data provided by Counties and other oversight agencies. The census block group data layer shows demographic information and drought risk information available at the block group level and water quality, domestic well, and state small information summarized by block group. The demographic information includes race/ethnicity data and median household income data, and the drought risk data is from the Department of Water Resources rural/self-supplied community scores. Other reference layers that can be overlain on the map include boundaries of priority areas in the Central Valley CV-SALTS program, Groundwater Sustainability Agency boundaries, and current groundwater level data from the Department of Water Resources (DWR).

# Summary of 2022 Updates

- Re-focus Aquifer Risk Map to section level data instead of census block group risk percentile scores.

- Change definition of “recent” results from 2 to 5 years.

- Incorporate water quality data from cleanup monitoring sites (GeoTracker data).

- Display drought risk data as Department of Water Resources (DWR) risk scores for self-supplied/rural communities and as current groundwater level conditions from DWR.

- Display race and ethnicity data and median household income (disadvantaged community status).

# Intended Use of this Analysis

The water quality risk ranking developed using this methodology are not intended to depict actual groundwater quality conditions at any given domestic supply well or small water system location. The purpose of this risk map analysis is to prioritize areas that may not meet primary drinking water standards for additional investigation and sampling efforts. The current lack of available domestic well and state small system water quality data makes it not possible to characterize the water quality for individual domestic wells and state small systems. The analysis described herein represents a best effort at using the available data to estimate water quality risk for domestic wells and state small systems.

# Unit of Analysis

Groundwater quality risk is summarized by square mile section. Water quality data also can be viewed as individual well points. State small system location data is available as point locations, and domestic well density information is available as both count per square mile and as count per census block group. The count of high risk, medium risk, low risk, and no data domestic wells and state small water systems are also summarized by census block group for comparison with demographic data and DWR drought risk scores.

# Methodology

## Data Processing

Water quality results from the Division of Drinking Water (DDW), the US Geological Survey (USGS)-Groundwater Ambient Monitoring and Assessment (GAMA) programs’ Priority Basin and Domestic Well Projects, the USGS-National Water Information System dataset, the Department of Water Resources (DWR), local groundwater monitoring projects, the Irrigated Lands Regulatory Program (AGLAND), and cleanup monitoring sites (GeoTracker)were included in this analysis. Results were only included if the well met the depth-filtering criteria developed in Domestic Well Needs Assessment project. Data from most chemical constituents with a Maximum Contaminant Level (MCL) are assessed, and several additional chemical constituents including hexavalent chromium, copper, lead, and N-Nitrosodimethylamine (NDMA) are included in the analysis as well. Water quality results were converted to an MCL Index to allow comparison between chemical constituents (see Appendix A for table with MCLs). A more detailed presentation of data collection, data standardization, and data filtering are outlined in the Needs Assessment Domestic Well Water Quality Tool White Paper. The R script used to download, process, and filter the water quality data is available on GitHub.

## Depth Filter

Most available groundwater quality data is sourced from public (municipal) supply wells. This is a result of California’s requirement for monitoring and reporting of groundwater from wells that are part of a water system that supplies water to 15 or more service connections. In contrast, domestic wells (any system that serves less than 5 connections) and state small water systems (5 – 14 connections) are not regulated by the state and therefore lack comprehensive data.

For many regions, municipal supply wells access a deeper portion of the groundwater resource when compared with domestic wells. This deeper groundwater is typically less affected by contaminants introduced at the ground surface than shallower groundwater. As a result, use of data from municipal wells would likely result in a systematic low bias for an estimate of the shallower groundwater typically accessed by domestic wells.

Staff developed a method to filter data that more likely represents shallower groundwater accessed by domestic wells, as summarized below.

Since well depth varies throughout the state, a domestic depth zone was defined numerically for each groundwater unit based on Total Completed Depth statistics from the OSWCR database. Based on well depth data in the OSCWR database, a well depth interval per groundwater unit was determined for wells classified as domestic and for wells classified as public (Figure 1). These well depth statistics were then compared to assess whether domestic and public well depth intervals overlap, indicating they access the same groundwater source. For groundwater units where the depth interval for public and domestic wells overlapped (or the public interval was shallower) water quality data from public wells was included in the analysis. For groundwater units where the depth interval for public wells was deeper than the depth interval for domestic wells water quality data from public wells was screened out of the analysis. For details on the maximum domestic well depth and the comparison of public and domestic wells for each groundwater unit, see Appendix B.

Figure 1. Numeric depth filter – based on average of section maximum/minimum well depths per Groundwater Unit. Wells with a known depth that fall within the “domestic well depth interval” are included in the analysis. Wells with a known depth that fall outside the “domestic well depth interval” are screened out of the analysis. For wells without a known depth - if the “public bottom” depth of a Groundwater Unit is shallower or within 10% of the “domestic bottom” depth, then wells classified as public are included in the analysis. If the “public bottom” depth of a Groundwater Unit is more than 10% deeper than the “domestic bottom” depth, then wells classified as public are screened out of the analysis.

Figure 2. Depth filter by well type (for wells with unknown depth). This map shows basins where domestic wells and public wells may be accessing similar groundwater depths (pink) and basins where domestic wells and public wells are accessing different groundwater depths (blue). For the basins show in pink, public wells were used as a proxy for domestic depth water quality.

Most wells with water quality data do not have well construction data (depth of well or screen interval). Wells with depth data were filtered based on their numeric well construction; wells without numeric construction data were filtered by well type.

Wells with known numeric depths

Staff used OSWCR Total Completed Depth section summary statistics to determine a “Domestic Bottom” and “Domestic Top” depth for each Groundwater Unit. The domestic well depth zone was defined as the range between “Domestic Bottom” depth and “Domestic Top” depth. For Group 1 wells, if the given depth of the well fell between the “Domestic Top” depth and the “Domestic Bottom” depth, water quality data from that well was included in the analysis.

Water quality data wells with unknown numeric depths

Staff used OSWCR well depth information to compare “Domestic Bottom” depth (defined above) to “Public Bottom” depth (defined below). If the “Public Bottom” depth for a given Groundwater Unit was shallower than the “Domestic Bottom” depth, or within 10% of “Domestic Bottom” depth (shallower or deeper), then it was considered reasonable to include data from public wells into the analysis for that Groundwater Unit. If the “Public Bottom” depth for a given Groundwater Unit was more than 10% deeper than the “Domestic Bottom” depth, water quality data from public wells was screened out of the analysis for that Groundwater Unit.

## De-clustering

Available water quality results were spatially and temporally de-clustered to square mile sections to account for differences in data sampling density within each section over space and time. This was conducted to prevent certain areas with a high density of wells and frequent sampling to achieve a disproportionate weighting to the overall risk characterization of an area. To expand the coverage of the water quality risk map, averaged, de-clustered data from sections that contain a well(s) that provide  water quality data (“source sections”) are projected onto neighboring sections that do not include a well providing water quality data.

Water quality data is assessed using two metrics - the long-term (20 year) average and all recent results (within 5 years). The temporal and spatial de-clustering methodology for each metric is outlined below, and is further described in the Domestic Well Needs Assessment White Paper:

### Long-term average

- Water quality results from each well for each chemical constituent are averaged per year (for the past 20 years).

- The results from step one is averaged per well.

- The results from step two are averaged for all the wells that lie within a section.

- For sections that do not contain a well with water quality data, the de-clustered data from step three is projected onto adjacent sections.

### Recent results

- All recent (within the past 5 years) results in a section are categorized as “under” (less than 80 percent of MCL), “close” (80 percent – 100 percent of MCL), or “over” (greater than MCL).

- The count of recent results in each category are summarized per square mile section for each constituent.

- For square mile sections that do not contain a well with recent water quality data, the results from step two is averaged for all adjacent sections.

## Water Quality Risk

Water quality data is summarized by square mile section using the methodology outlined above. For each square mile section, several metrics are reported and the sections are then grouped into “high risk”, “medium risk”, “low risk”, or “no data”. This water quality risk is then combined with the density of domestic well records and the location of state small water system to identify the number and location of wells and systems at risk.

Table 1. Description of data available per square mile section.

| Column | Name | Description |
| --- | --- | --- |
| MTRS | Public Land Survey Section ID | Unique section identifier (square mile section) |
| SRF1 | Count of contaminants above MCL | Count of contaminants with long-term average or recent results above MCL |
| SRF2 | Count of contaminants close to MCL | Count of contaminants long-term average or recent results between 80-100% of MCL |
| SRF3 | Magnitude of contamination | Average magnitude of long-term average or recent results for contaminants above MCL (reported as MCL index) |
| SL1 | List of contaminants above MCL | List of contaminants above MCL |
| SL2 | List of contaminants close to MCL | List of contaminants between 80-100% of MCL |
| WQriskbins | Water Quality Risk | Water quality risk based on comparison of long-term average and recent results to MCL |
| dw_1970na_desna | Domestic Well Record Count | Total number of domestic wells in section (OSWCR), excluding any domestic wells drilled prior to 1970 and any records that include the word “destruction” |
| ssws_count | State Small Water System Count (RCAC) | Total number of state small water systems in section (RCAC) |
| ssws_ddw | State Small Water System Count (DDW) | Total number of state small water systems in section (DDW) |

Table 2. Definition of water quality risk bins.

| Water Quality Risk Bin | Description |
| --- | --- |
| High Risk | Long-term average or recent results are above MCL for one or more contaminants (highest risk contaminant represents section) |
| Medium Risk | Long-term average or recent results are between 80-100% of MCL for one or more contaminants (highest risk contaminant represents section) |
| Low Risk | Long-term average or recent results are below 80% of MCL for all sampled contaminants |
| No Data | No water quality data available that meets depth- or time-filtering criteria |

### Areas with No Available Water Quality Data

Approximately 22% (68,192) of domestic well records and 11% (141) of state small water systems are in areas with no available water quality data.

### Individual Chemical Constituents

Single-chemical constituent layers are available as square mile section data for Nitrate, Arsenic, 1,2,3-Trichloropropane, Hexavalent Chromium, and Uranium. These layers display the long-term average and the count of recent results over, close to, and under the MCL per square mile section for a single chemical constituent.

### Summary by census block group

To facilitate comparison with data reported at the census block group level, including demographic data and DWR drought risk scores the 2022 Aquifer Risk Map results for domestic wells and state small water systems are summarized by census block groups. The number of “high risk”, “medium risk”, “low risk” and “no data” domestic wells and state small systems are calculated, as is the percentage of “high risk”, “medium risk”, “low risk”, and “no data” sections within the block group. Unlike last year, domestic wells are now assigned to a block group by percent area, instead of counts being duplicated across block group boundaries where sections intersected multiple block groups.

Table 3. Example of data summarized and displayed for each census block group.

| GEOID | 060670092012 |
| --- | --- |
| County | Sacramento County |
| High-Risk Domestic Wells | 34.67 |
| Total Domestic Wells | 153.21 |
| High-Risk State Small Water Systems | 0 |
| Total State Small Water Systems | 0 |
| Median Household Income (2018 ACS) | 51354 |
| DAC/SDAC status | DAC |
| DWR Drought Risk Score | 48.85 |
| Total Population | 1036 |
| Hispanic | 301 |
| White | 465 |
| Black | 39 |
| American Indian/Native American | 7 |
| Asian | 144 |
| Native Hawaiian/Pacific Islander | 16 |
| Other/Multiple | 64 |

# Summarized Results

Water quality risk for domestic wells and state small systems are summarized statewide and by county.

Figure 3. Summary of 2022 Aquifer Risk Map statewide results (domestic wells and state small water systems).

Figure 4. Summary of 2022 Aquifer Risk Map results for state small water system by county. Only counties with > 1 high risk state small water system are shown.

Figure 5. Summary of 2022 Aquifer Risk Map results for domestic wells by county. Only counties with > 100 high risk domestic wells are shown.

# Comparison of 2021 and 2022 Aquifer Risk Map Results

## Comparison with ILRP domestic well data

The estimated nitrate water quality risk from the 2021 Aquifer Risk Map was compared to actual domestic well nitrate data collected through the Irrigated Lands Regulatory Program (ILRP) from September 2020 through May 2021 to assess the accuracy of the 2021 risk map. These ILRP nitrate results were not used to calculate the 2021 Aquifer Risk Map water quality risk estimates. As a result, these ILRP results provide a control group for comparing to the predicted nitrate water quality in the 2021 Aquifer Risk Map. For 74% (1,267) of ILRP samples the 2021 Aquifer Risk Map accurately estimated the ILRP nitrate results or was close to estimating the ILRP nitrate results. For 15% (258) of ILRP samples the 2021 Aquifer Risk Map estimated nitrate to be “high risk” (> 10mg/L) and the sampled result was “low risk” (<= 8 mg/L) and for 11% (185) of ILRP samples the 2021 Aquifer Risk Map estimated nitrate to be “low risk” (<= 8 mg/L) and the sampled results was “high risk” (> 10mg/L).

## Analysis of 2022 Methodology Changes

Domestic well data sampled in 2020 through the Irrigated Lands Regulatory Program and other datasets meant that the data coverage of the Aquifer Risk Map improved from 2021 to 2022. In the 2021 map, 26% of domestic well records and 16% of state small water systems were in areas with no water quality data. In the 2022 map, 22% of domestic well records and 11% of state small water systems are in areas with no water quality data. The updated methodology meant that more domestic wells and state small systems are in “high risk” areas in 2022 compared to 2021. However, without methodology changes there is almost no change in the percentage of “high risk” domestic wells and there is a slight decrease in the percentage of “high risk” state small water systems.

Figure 6. Comparison of water quality risk for domestic wells for the 2021 and 2022 Aquifer Risk Maps. The "2021 ARM" column shows the domestic well risk for the 2021 Aquifer Risk Map. The "2022 Arm (all proposed updates)" column shows the domestic well risk for the 2022 Aquifer Risk Map, including all methodology changes. The “2022 ARM (no updates)” column shows the domestic well risk for using 2022 data, but 2021 methodology. This shows that the change from 2021 to 2022 would have been if there were no methodology changes.

Figure 7. Comparison of water quality risk for state small water systems for the 2021 and 2022 Aquifer Risk Maps. The "2021 ARM" column shows the state small water system risk for the 2021 Aquifer Risk Map. The "2022 Arm (all proposed updates)" column shows the state small water system risk for the 2022 Aquifer Risk Map, including all methodology changes. The “2022 ARM (no updates)” column shows the state small water system risk using 2022 data, but 2021 methodology. This shows that the change from 2021 to 2022 would have been if there were no methodology changes. Note that this analysis was done on RCAC state small water system counts, to remain consistent with what was used in 2021. In the final 2022 Aquifer Risk Map, state small water system locations from DDW were used, resulting in slightly different statewide totals.

# Appendix A – Chemical Abbreviations and Comparison Concentrations

| Chemical Abbreviation (Web Tool) | Chemical Name | Units | Comparison Concentration Value | Comparison Concentration Type |
| --- | --- | --- | --- | --- |
| 24D | 2,4-Dichlorophenoxyacetic acid (2,4 D) | UG/L | 70 | MCL |
| AL | Aluminum | UG/L | 1000 | MCL |
| ALACL | Alachlor | UG/L | 2 | MCL |
| ALPHA | Gross Alpha radioactivity | pCi/L | 15 | MCL |
| AS | Arsenic | UG/L | 10 | MCL |
| ATRAZINE | Atrazine | UG/L | 1 | MCL |
| BA | Barium | MG/L | 1 | MCL |
| BDCME | Bromodichloromethane (THM) | UG/L | 80 | MCL |
| BE | Beryllium | UG/L | 4 | MCL |
| BETA | Gross beta | pCi/L | 50 | MCL |
| BHCGAMMA | Lindane (Gamma-BHC) | UG/L | 0.2 | MCL |
| BIS2EHP | Di(2-ethylhexyl)phthalate (DEHP) | UG/L | 4 | MCL |
| BRO3 | Bromate | UG/L | 10 | MCL |
| BTZ | Bentazon | UG/L | 18 | MCL |
| BZ | Benzene | UG/L | 1 | MCL |
| BZAP | Benzo(a)pyrene | UG/L | 0.2 | MCL |
| BZME | Toluene | UG/L | 150 | MCL |
| CD | Cadmium | UG/L | 5 | MCL |
| CHLORDANE | Chlordane | UG/L | 0.1 | MCL |
| CHLORITE | Chlorite | MG/L | 1 | MCL |
| CLBZ | Chlorobenzene | UG/L | 70 | MCL |
| CN | Cyanide (CN) | UG/L | 150 | MCL |
| CR | Chromium | UG/L | 50 | MCL |
| CR6 | Chromium, Hexavalent (Cr6) | UG/L | 10 | Temporary comparison level* |
| CRBFN | Carbofuran | UG/L | 18 | MCL |
| CTCL | Carbon Tetrachloride | UG/L | 0.5 | MCL |
| CU | Copper | MG/L | 1.3 | Action Level |
| DALAPON | Dalapon | UG/L | 200 | MCL |
| DBCME | Dibromochloromethane (THM) | UG/L | 80 | MCL |
| DBCP | 1,2-Dibromo-3-chloropropane (DBCP) | UG/L | 0.2 | MCL |
| DCA11 | 1,1-Dichloroethane (1,1 DCA) | UG/L | 5 | MCL |
| DCA12 | 1,2 Dichloroethane (1,2 DCA) | UG/L | 0.5 | MCL |
| DCBZ12 | 1,2 Dichlorobenzene (1,2-DCB) | UG/L | 600 | MCL |
| DCBZ14 | 1,4-Dichlorobenzene (p-DCB) | UG/L | 5 | MCL |
| DCE11 | 1,1 Dichloroethylene (1,1 DCE) | UG/L | 6 | MCL |
| DCE12C | cis-1,2 Dichloroethylene | UG/L | 6 | MCL |
| DCE12T | trans-1,2, Dichloroethylene | UG/L | 10 | MCL |
| DCMA | Dichloromethane (Methylene Chloride) | UG/L | 5 | MCL |
| DCP13 | 1,3 Dichloropropene | UG/L | 0.5 | MCL |
| DCPA12 | 1,2 Dichloropropane (1,2 DCP) | UG/L | 5 | MCL |
| DINOSEB | Dinoseb | UG/L | 7 | MCL |
| DIQUAT | Diquat | UG/L | 20 | MCL |
| DOA | Di(2-ethylhexyl)adipate | MG/L | 0.4 | MCL |
| EBZ | Ethylbenzene | UG/L | 300 | MCL |
| EDB | 1,2 Dibromoethane (EDB) | UG/L | 0.05 | MCL |
| ENDOTHAL | Endothall | UG/L | 100 | MCL |
| ENDRIN | Endrin | UG/L | 2 | MCL |
| F | Fluoride | MG/L | 2 | MCL |
| FC11 | Trichlorofluoromethane (Freon 11) | UG/L | 150 | MCL |
| FC113 | 1,1,2-Trichloro-1,2,2-Trifluoroethane (Freon 113) | MG/L | 1.2 | MCL |
| GLYP | Glyphosate (Round-up) | UG/L | 700 | MCL |
| H-3 | Tritium | pCi/L | 20000 | MCL |
| HCCP | Hexachlorocyclopentadiene | UG/L | 50 | MCL |
| HCLBZ | Hexachlorobenzene (HCB) | UG/L | 1 | MCL |
| HEPTACHLOR | Heptachlor | UG/L | 0.01 | MCL |
| HEPT-EPOX | Heptachlor Epoxide | UG/L | 0.01 | MCL |
| HG | Mercury | UG/L | 2 | MCL |
| MOLINATE | Molinate | UG/L | 20 | MCL |
| MTBE | MTBE (Methyl-tert-butyl ether) | UG/L | 13 | MCL |
| MTXYCL | Methoxychlor | UG/L | 30 | MCL |
| NI | Nickel | UG/L | 100 | MCL |
| NNSM | N-Nitrosodimethylamine (NDMA) | UG/L | 0.01 | NL |
| NO2 | Nitrite as N | MG/L | 1 | MCL |
| NO3N | Nitrate as N | MG/L | 10 | MCL |
| OXAMYL | Oxamyl | UG/L | 50 | MCL |
| PB | Lead | UG/L | 15 | Action Level |
| PCA | 1,1,2,2 Tetrachloroethane (PCA) | UG/L | 1 | MCL |
| PCATE | Perchlorate | UG/L | 6 | MCL |
| PCB1016 | Polychlorinated Biphenyls (PCBs) | UG/L | 0.5 | MCL |
| PCE | Tetrachloroethene (PCE) | UG/L | 5 | MCL |
| PCP | Pentachlorophenol (PCP) | UG/L | 1 | MCL |
| PICLORAM | Picloram | MG/L | 0.5 | MCL |
| RA-226 | Radium 226 | pCi/L | 5 | MCL |
| RA-228 | Radium 228 | pCi/L | 5 | MCL |
| SB | Antimony | UG/L | 6 | MCL |
| SE | Selenium | UG/L | 50 | MCL |
| SILVEX | 2,4,5-TP (Silvex) | UG/L | 50 | MCL |
| SIMAZINE | Simazine | UG/L | 4 | MCL |
| SR-90 | Strontium 90 | pCi/L | 8 | MCL |
| STY | Styrene | UG/L | 100 | MCL |
| TBME | Bromoform (THM) | UG/L | 80 | MCL |
| TCA111 | 1,1,1-Trichloroethane | UG/L | 200 | MCL |
| TCA112 | 1,1,2-Trichloroethane | UG/L | 5 | MCL |
| TCB124 | 1,2,4- Trichlorobenzene (1,2,4 TCB) | UG/L | 5 | MCL |
| TCDD2378** | 2,3,7,8-Tetrachlorodibenzodioxin (Dioxin) | UG/L | 3.00E-05 | MCL |
| TCE | Trichloroethene (TCE) | UG/L | 5 | MCL |
| TCLME | Chloroform (THM) | UG/L | 80 | MCL |
| TCPR123 | 1,2,3-Trichloropropane (1,2,3 TCP) | UG/L | 0.005 | MCL |
| THIOBENCARB | Thiobencarb | UG/L | 70 | MCL |
| THM | Total Trihalomethanes | UG/L | 80 | MCL |
| TL | Thallium | UG/L | 2 | MCL |
| TOXAP | Toxaphene | UG/L | 3 | MCL |
| U | Uranium | pCi/L | 20 | MCL |
| VC | Vinyl Chloride | UG/L | 0.5 | MCL |
| XYLENES | Xylenes (total) | UG/L | 1750 | MCL |

*Since there is currently no MCL for Hexavalent Chromium (CR6), a temporary comparison value was to remain consistent with the risk assessment for public water systems.

**No data for 2,3,7,8-Tetrachlorodibenzodioxin (Dioxin) was available for this analysis, because there are no samples from wells that met our depth/time criteria.

# Appendix B. Depth Filter

This table lists the depth filter output for each groundwater unit in California. The table shows the ID, name, maximum domestic depth (in feet) and whether that groundwater unit has domestic and public wells at similar depths. The numeric value in the third column indicates the domestic depth maximum cutoff – only wells with shallower depths are used to estimate domestic/state small water quality. A “no” in the final column indicates that domestic and public wells are accessing different groundwater depths, and public wells are not used to estimate domestic/state small water quality when well depth is unknown. A “yes” in the final column indicates that domestic and public wells are accessing similar groundwater depths, and public wells are used to estimated domestic/state small water quality when well depth is unknown.

| Groundwater Unit ID | Groundwater Unit Name | Max Domestic Well Depth (ft) | Use public wells as proxy for domestic? |
| --- | --- | --- | --- |
| 1-1 | SMITH RIVER PLAIN | 304 | yes |
| 1-10 | EEL RIVER VALLEY | 606 | no |
| 1-11 | COVELO ROUND VALLEY | 429 | no |
| 1-12 | LAYTONVILLE VALLEY | 659 | no |
| 1-13 | LITTLE LAKE VALLEY | 558 | yes |
| 1-14 | LOWER KLAMATH RIVER VALLEY | 281 | yes |
| 1-15 | HAPPY CAMP TOWN AREA | 467 | yes |
| 1-16 | SEIAD VALLEY | 747 | yes |
| 1-17 | BRAY TOWN AREA | 876 | yes |
| 1-18 | RED ROCK VALLEY | 503 | yes |
| 1-19 | ANDERSON VALLEY | 530 | no |
| 1-2 | KLAMATH RIVER VALLEY | 621 | no |
| 1-20 | GARCIA RIVER VALLEY | 363 | no |
| 1-21 | FORT BRAGG TERRACE AREA | 560 | yes |
| 1-22 | FAIRCHILD SWAMP VALLEY | NA | NA |
| 1-25 | PRAIRIE CREEK AREA | 178 | no |
| 1-26 | REDWOOD CREEK AREA | 195 | yes |
| 1-27 | BIG LAGOON AREA | 437 | yes |
| 1-28 | MATTOLE RIVER VALLEY | 228 | no |
| 1-29 | HONEYDEW TOWN AREA | 323 | no |
| 1-3 | BUTTE VALLEY | 959 | no |
| 1-30 | PEPPERWOOD TOWN AREA | 417 | no |
| 1-31 | WEOTT TOWN AREA | 367 | yes |
| 1-32 | GARBERVILLE TOWN AREA | 663 | no |
| 1-33 | LARABEE VALLEY | 484 | yes |
| 1-34 | DINSMORES TOWN AREA | 435 | no |
| 1-35 | HYAMPOM VALLEY | 431 | yes |
| 1-36 | HETTENSHAW VALLEY | 414 | yes |
| 1-37 | COTTONEVA CREEK VALLEY | 554 | yes |
| 1-38 | LOWER LAYTONVILLE VALLEY | 420 | yes |
| 1-39 | BRANSCOMB TOWN AREA | 487 | no |
| 1-4 | SHASTA VALLEY | 748 | yes |
| 1-40 | TEN MILE RIVER VALLEY | 563 | yes |
| 1-41 | LITTLE VALLEY | 393 | yes |
| 1-42 | SHERWOOD VALLEY | 416 | no |
| 1-43 | WILLIAMS VALLEY | 646 | yes |
| 1-44 | EDEN VALLEY | NA | NA |
| 1-45 | BIG RIVER VALLEY | 466 | yes |
| 1-46 | NAVARRO RIVER VALLEY | 484 | yes |
| 1-48 | GRAVELLY VALLEY | 643 | yes |
| 1-49 | ANNAPOLIS OHLSON RANCH FM HIGHLANDS | 510 | yes |
| 1-5 | SCOTT RIVER VALLEY | 516 | no |
| 1-50 | KNIGHTS VALLEY | 759 | yes |
| 1-51 | POTTER VALLEY | 562 | yes |
| 1-52 | UKIAH VALLEY | 715 | no |
| 1-53 | SANEL VALLEY | 450 | no |
| 1-54 | ALEXANDER VALLEY | 983 | yes |
| 1-55 | SANTA ROSA VALLEY | 1365 | yes |
| 1-56 | McDOWELL VALLEY | 530 | no |
| 1-57 | BODEGA BAY AREA | 429 | yes |
| 1-59 | WILSON GROVE FORMATION HIGHLANDS | 1216 | yes |
| 1-6 | HAYFORK VALLEY | 603 | yes |
| 1-60 | LOWER RUSSIAN RIVER VALLEY | 719 | yes |
| 1-61 | FORT ROSS TERRACE DEPOSITS | 569 | no |
| 1-62 | WILSON POINT AREA | 412 | yes |
| 1-7 | HOOPA VALLEY | 141 | no |
| 1-8 | MAD RIVER VALLEY | 403 | yes |
| 1-9 | EUREKA PLAIN | 512 | no |
| 2-1 | PETALUMA VALLEY | 996 | yes |
| 2-10 | LIVERMORE VALLEY | 872 | no |
| 2-11 | SUNOL VALLEY | 864 | yes |
| 2-19 | KENWOOD VALLEY | 1255 | yes |
| 2-2 | NAPA-SONOMA VALLEY | 1277 | yes |
| 2-22 | HALF MOON BAY TERRACE | 925 | yes |
| 2-24 | SAN GREGORIO VALLEY | 466 | yes |
| 2-26 | PESCADERO VALLEY | 589 | yes |
| 2-27 | SAND POINT AREA | 758 | yes |
| 2-28 | ROSS VALLEY | 685 | yes |
| 2-29 | SAN RAFAEL VALLEY | 522 | yes |
| 2-3 | SUISUN-FAIRFIELD VALLEY | 624 | yes |
| 2-30 | NOVATO VALLEY | 830 | no |
| 2-31 | ARROYO DEL HAMBRE VALLEY | 520 | yes |
| 2-32 | VISITACION VALLEY | NA | NA |
| 2-33 | ISLAIS VALLEY | 857 | no |
| 2-35 | WESTSIDE | 851 | no |
| 2-36 | SAN PEDRO VALLEY | 515 | yes |
| 2-37 | SOUTH SAN FRANCISCO | NA | NA |
| 2-38 | LOBOS | NA | NA |
| 2-39 | MARINA | NA | NA |
| 2-4 | PITTSBURG PLAIN | 463 | no |
| 2-40 | DOWNTOWN | 626 | yes |
| 2-5 | CLAYTON VALLEY | 839 | yes |
| 2-6 | YGNACIO VALLEY | 602 | yes |
| 2-7 | SAN RAMON VALLEY | 806 | yes |
| 2-8 | CASTRO VALLEY | 427 | yes |
| 2-9 | SANTA CLARA VALLEY | 1060 | no |
| 3-1 | SOQUEL VALLEY | 1347 | no |
| 3-12 | SANTA MARIA | 1133 | no |
| 3-13 | CUYAMA VALLEY | 1175 | no |
| 3-14 | SAN ANTONIO CREEK VALLEY | 1320 | no |
| 3-15 | SANTA YNEZ RIVER VALLEY | 1350 | yes |
| 3-16 | GOLETA | 1225 | no |
| 3-17 | SANTA BARBARA | 1012 | yes |
| 3-18 | CARPINTERIA | 1409 | yes |
| 3-19 | CARRIZO PLAIN | 648 | yes |
| 3-2 | PAJARO VALLEY | 1134 | yes |
| 3-20 | ANO NUEVO AREA | 427 | no |
| 3-21 | SANTA CRUZ PURISIMA FORMATION | 1288 | no |
| 3-22 | SANTA ANA VALLEY | 838 | yes |
| 3-23 | UPPER SANTA ANA VALLEY | 540 | yes |
| 3-24 | QUIEN SABE VALLEY | NA | NA |
| 3-25 | TRES PINOS VALLEY | 663 | yes |
| 3-26 | WEST SANTA CRUZ TERRACE | 1000 | yes |
| 3-27 | SCOTTS VALLEY | 1324 | yes |
| 3-28 | SAN BENITO RIVER VALLEY | 630 | no |
| 3-29 | DRY LAKE VALLEY | 873 | yes |
| 3-3 | GILROY-HOLLISTER VALLEY | 955 | yes |
| 3-30 | BITTER WATER VALLEY | 729 | yes |
| 3-31 | HERNANDEZ VALLEY | 318 | yes |
| 3-32 | PEACH TREE VALLEY | 89 | yes |
| 3-33 | SAN CARPOFORO VALLEY | 1187 | yes |
| 3-34 | ARROYO DE LA CRUZ VALLEY | 555 | yes |
| 3-35 | SAN SIMEON VALLEY | NA | NA |
| 3-36 | SANTA ROSA VALLEY | 756 | yes |
| 3-37 | VILLA VALLEY | 545 | yes |
| 3-38 | CAYUCOS VALLEY | NA | NA |
| 3-39 | OLD VALLEY | 670 | yes |
| 3-4.1 | 180/400 FOOT AQUIFER; EAST SIDE AQUIFER; SEASIDE AREA; LANGLEY AREA; CORRAL DE TIERRA AREA | 1309 | yes |
| 3-4.2 | FOREBAY AQUIFER; UPPER VALLEY AQUIFER | 937 | no |
| 3-4.06 | PASO ROBLES AREA | 1118 | yes |
| 3-40 | TORO VALLEY | NA | NA |
| 3-41 | MORRO VALLEY | 529 | yes |
| 3-42 | CHORRO VALLEY | 959 | yes |
| 3-43 | RINCONADA VALLEY | 680 | yes |
| 3-44 | POZO VALLEY | 1064 | yes |
| 3-45 | HUASNA VALLEY | 607 | yes |
| 3-46 | RAFAEL VALLEY | NA | NA |
| 3-47 | BIG SPRING AREA | NA | NA |
| 3-49 | MONTECITO | 1437 | yes |
| 3-5 | CHOLAME VALLEY | 516 | yes |
| 3-50 | FELTON AREA | 1267 | yes |
| 3-51 | MAJORS CREEK | NA | NA |
| 3-52 | NEEDLE ROCK POINT | 596 | yes |
| 3-53 | FOOTHILL | 1411 | yes |
| 3-6 | LOCKWOOD VALLEY | 833 | yes |
| 3-7 | CARMEL VALLEY | 1515 | yes |
| 3-8 | LOS OSOS VALLEY | 476 | no |
| 3-9 | SAN LUIS OBISPO VALLEY | 795 | yes |
| 4-1 | UPPER OJAI VALLEY | 940 | yes |
| 4-10 | CONEJO | 1413 | yes |
| 4-11 | COASTAL PLAIN OF LOS ANGELES | 1596 | no |
| 4-12 | SAN FERNANDO VALLEY | 891 | no |
| 4-13 | SAN GABRIEL VALLEY | 1338 | no |
| 4-15 | TIERRA REJADA | 1029 | yes |
| 4-16 | HIDDEN VALLEY | 1225 | yes |
| 4-17 | LOCKWOOD VALLEY | 747 | no |
| 4-18 | HUNGRY VALLEY | 503 | no |
| 4-19 | THOUSAND OAKS AREA | 397 | no |
| 4-2 | OJAI VALLEY | 1038 | yes |
| 4-20 | RUSSELL VALLEY | 519 | yes |
| 4-22 | MALIBU VALLEY | 1513 | yes |
| 4-23 | RAYMOND | 960 | no |
| 4-3 | VENTURA RIVER VALLEY | 1086 | yes |
| 4-4 | SANTA CLARA RIVER VALLEY | 1586 | no |
| 4-5 | ACTON VALLEY | 1206 | yes |
| 4-6 | PLEASANT VALLEY | 1682 | no |
| 4-7 | ARROYO SANTA ROSA VALLEY | 1576 | yes |
| 4-8 | LAS POSAS VALLEY | 2068 | yes |
| 4-9 | SIMI VALLEY | 890 | yes |
| 5-1 | GOOSE LAKE | 599 | yes |
| 5-10 | AMERICAN VALLEY | 677 | no |
| 5-11 | MOHAWK VALLEY | 1047 | yes |
| 5-12 | SIERRA VALLEY | 850 | no |
| 5-13 | UPPER LAKE VALLEY | 507 | yes |
| 5-14 | SCOTTS VALLEY | 560 | yes |
| 5-15 | BIG VALLEY | 740 | yes |
| 5-16 | HIGH VALLEY | 610 | no |
| 5-17 | BURNS VALLEY | 640 | yes |
| 5-18 | COYOTE VALLEY | 658 | yes |
| 5-19 | COLLAYOMI VALLEY | 558 | no |
| 5-2 | ALTURAS AREA | 844 | yes |
| 5-20 | BERRYESSA VALLEY | 717 | yes |
| 5-21.50 | RED BLUFF | 682 | no |
| 5-21.51 | CORNING | 572 | no |
| 5-21.52 | COLUSA | 681 | no |
| 5-21.53 | BEND | 821 | no |
| 5-21.54 | ANTELOPE | 504 | no |
| 5-21.55 | DYE CREEK | 315 | yes |
| 5-21.56 | LOS MOLINOS | 368 | no |
| 5-21.57 | VINA | 641 | no |
| 5-21.58 | WEST BUTTE | 637 | no |
| 5-21.59 | EAST BUTTE | 567 | no |
| 5-21.60 | NORTH YUBA | 799 | yes |
| 5-21.61 | SOUTH YUBA | 706 | no |
| 5-21.62 | SUTTER | 523 | no |
| 5-21.63 | WEST SUTTER | 515 | no |
| 5-21.64 | NORTH AMERICAN | 709 | no |
| 5-21.65 | SOUTH AMERICAN | 686 | no |
| 5-21.66 | SOLANO | 754 | no |
| 5-21.67 | YOLO | 986 | no |
| 5-21.68 | CAPAY VALLEY | 578 | no |
| 5-22.01 | EASTERN SAN JOAQUIN | 840 | no |
| 5-22.02 | MODESTO | 778 | yes |
| 5-22.03 | TURLOCK | 649 | yes |
| 5-22.04 | MERCED | 911 | no |
| 5-22.05 | CHOWCHILLA | 899 | no |
| 5-22.06 | MADERA | 963 | no |
| 5-22.07 | DELTA-MENDOTA | 700 | no |
| 5-22.08 | KINGS | 828 | no |
| 5-22.09 | WESTSIDE | 1970 | yes |
| 5-22.10 | PLEASANT VALLEY | 1500 | no |
| 5-22.11 | KAWEAH | 732 | no |
| 5-22.12 | TULARE LAKE | 1353 | no |
| 5-22.13 | TULE | 926 | no |
| 5-22.14 | KERN COUNTY | 1667 | yes |
| 5-22.15 | TRACY | 789 | no |
| 5-22.16 | COSUMNES | 757 | no |
| 5-23 | PANOCHE VALLEY | 665 | no |
| 5-25 | KERN RIVER VALLEY | 886 | yes |
| 5-26 | WALKER BASIN CREEK VALLEY | 771 | yes |
| 5-27 | CUMMINGS VALLEY | 902 | yes |
| 5-28 | TEHACHAPI VALLEY WEST | 1219 | yes |
| 5-29 | CASTAC LAKE VALLEY | 1017 | yes |
| 5-3 | JESS VALLEY | 630 | yes |
| 5-30 | LOWER LAKE VALLEY | 492 | no |
| 5-31 | LONG VALLEY | 432 | yes |
| 5-35 | MCCLOUD AREA | 559 | yes |
| 5-36 | ROUND VALLEY | 611 | yes |
| 5-37 | TOAD WELL AREA | NA | NA |
| 5-38 | PONDOSA TOWN AREA | NA | NA |
| 5-4 | BIG VALLEY | 871 | yes |
| 5-40 | HOT SPRINGS VALLEY | 733 | yes |
| 5-41 | EGG LAKE VALLEY | 750 | yes |
| 5-43 | ROCK PRAIRIE VALLEY | 722 | yes |
| 5-44 | LONG VALLEY | NA | NA |
| 5-45 | CAYTON VALLEY | NA | NA |
| 5-46 | LAKE BRITTON AREA | 679 | yes |
| 5-47 | GOOSE VALLEY | 424 | yes |
| 5-48 | BURNEY CREEK VALLEY | 499 | yes |
| 5-49 | DRY BURNEY CREEK VALLEY | 860 | yes |
| 5-5 | FALL RIVER VALLEY | 726 | yes |
| 5-50 | NORTH FORK BATTLE CREEK | 807 | yes |
| 5-51 | BUTTE CREEK VALLEY | NA | NA |
| 5-52 | GRAYS VALLEY | NA | NA |
| 5-53 | DIXIE VALLEY | NA | NA |
| 5-54 | ASH VALLEY | 665 | yes |
| 5-56 | YELLOW CREEK VALLEY | 623 | yes |
| 5-57 | LAST CHANCE CREEK VALLEY | 855 | yes |
| 5-58 | CLOVER VALLEY | 432 | yes |
| 5-59 | GRIZZLY VALLEY | 773 | yes |
| 5-6 | REDDING AREA | 688 | no |
| 5-60 | HUMBUG VALLEY | 968 | no |
| 5-61 | CHROME TOWN AREA | 511 | yes |
| 5-62 | ELK CREEK AREA | 310 | yes |
| 5-63 | STONYFORD TOWN AREA | 291 | no |
| 5-64 | BEAR VALLEY | 600 | yes |
| 5-65 | LITTLE INDIAN VALLEY | 529 | no |
| 5-66 | CLEAR LAKE CACHE FORMATION | 586 | no |
| 5-68 | POPE VALLEY | 844 | yes |
| 5-69 | YOSEMITE VALLEY | 905 | yes |
| 5-7 | LAKE ALMANOR VALLEY | 160 | no |
| 5-70 | LOS BANOS CREEK VALLEY | NA | NA |
| 5-71 | VALLECITOS CREEK VALLEY | 660 | yes |
| 5-8 | MOUNTAIN MEADOWS VALLEY | 574 | yes |
| 5-82 | CUDDY CANYON VALLEY | 806 | yes |
| 5-83 | CUDDY RANCH AREA | 1139 | yes |
| 5-84 | CUDDY VALLEY | 994 | yes |
| 5-85 | MIL POTRERO AREA | 725 | no |
| 5-86 | JOSEPH CREEK | 718 | yes |
| 5-87 | MIDDLE FORK FEATHER RIVER | 978 | yes |
| 5-88 | STONY GORGE RESERVOIR | 610 | yes |
| 5-89 | SQUAW FLAT | NA | NA |
| 5-9 | INDIAN VALLEY | 694 | no |
| 5-90 | FUNKS CREEK | 166 | yes |
| 5-91 | ANTELOPE CREEK | 570 | yes |
| 5-92 | BLANCHARD VALLEY | 225 | yes |
| 5-93 | NORTH FORK CACHE CREEK | NA | NA |
| 5-94 | MIDDLE CREEK | 480 | yes |
| 5-95 | MEADOW VALLEY | 776 | yes |
| 6-1 | SURPRISE VALLEY | 754 | no |
| 6-10 | ADOBE LAKE VALLEY | 448 | yes |
| 6-100 | SECRET VALLEY | 997 | yes |
| 6-101 | BULL FLAT | NA | NA |
| 6-104 | LONG VALLEY | 699 | yes |
| 6-105 | SLINKARD VALLEY | NA | NA |
| 6-106 | LITTLE ANTELOPE VALLEY | 1090 | yes |
| 6-107 | SWEETWATER FLAT | NA | NA |
| 6-11 | LONG VALLEY | 822 | yes |
| 6-12 | OWENS VALLEY | 746 | no |
| 6-13 | BLACK SPRINGS VALLEY | NA | NA |
| 6-14 | FISH LAKE VALLEY | 984 | yes |
| 6-15 | DEEP SPRINGS VALLEY | 732 | no |
| 6-16 | EUREKA VALLEY | NA | NA |
| 6-17 | SALINE VALLEY | 640 | yes |
| 6-18 | DEATH VALLEY | 744 | yes |
| 6-19 | WINGATE VALLEY | NA | NA |
| 6-2 | MADELINE PLAINS | 598 | no |
| 6-20 | MIDDLE AMARGOSA VALLEY | 704 | yes |
| 6-21 | LOWER KINGSTON VALLEY | NA | NA |
| 6-22 | UPPER KINGSTON VALLEY | 649 | no |
| 6-23 | RIGGS VALLEY | 530 | no |
| 6-24 | RED PASS VALLEY | 730 | no |
| 6-25 | BICYCLE VALLEY | NA | NA |
| 6-26 | AVAWATZ VALLEY | NA | NA |
| 6-27 | LEACH VALLEY | NA | NA |
| 6-28 | PAHRUMP VALLEY | 426 | yes |
| 6-29 | MESQUITE VALLEY | 924 | yes |
| 6-3 | WILLOW CREEK VALLEY | 785 | yes |
| 6-30 | IVANPAH VALLEY | 1253 | yes |
| 6-31 | KELSO VALLEY | 775 | yes |
| 6-32 | BROADWELL VALLEY | NA | NA |
| 6-33 | SODA LAKE VALLEY | 540 | no |
| 6-34 | SILVER LAKE VALLEY | 770 | yes |
| 6-35 | CRONISE VALLEY | 610 | no |
| 6-36 | LANGFORD VALLEY | 1177 | yes |
| 6-37 | COYOTE LAKE VALLEY | 631 | no |
| 6-38 | CAVES CANYON VALLEY | 535 | no |
| 6-4 | HONEY LAKE VALLEY | 807 | no |
| 6-40 | LOWER MOJAVE RIVER VALLEY | 540 | no |
| 6-41 | MIDDLE MOJAVE RIVER VALLEY | 750 | yes |
| 6-42 | UPPER MOJAVE RIVER VALLEY | 1051 | no |
| 6-43 | EL MIRAGE VALLEY | 1008 | no |
| 6-44 | ANTELOPE VALLEY | 985 | no |
| 6-45 | TEHACHAPI VALLEY EAST | 925 | yes |
| 6-46 | FREMONT VALLEY | 842 | no |
| 6-47 | HARPER VALLEY | 704 | no |
| 6-48 | GOLDSTONE VALLEY | NA | NA |
| 6-49 | SUPERIOR VALLEY | 1015 | yes |
| 6-5 | TAHOE VALLEY | 584 | no |
| 6-50 | CUDDEBACK VALLEY | 713 | yes |
| 6-51 | PILOT KNOB VALLEY | 1070 | yes |
| 6-52 | SEARLES VALLEY | 933 | yes |
| 6-53 | SALT WELLS VALLEY | 750 | yes |
| 6-54 | INDIAN WELLS VALLEY | 910 | no |
| 6-55 | COSO VALLEY | NA | NA |
| 6-56 | ROSE VALLEY | 886 | yes |
| 6-57 | DARWIN VALLEY | NA | NA |
| 6-58 | PANAMINT VALLEY | NA | NA |
| 6-6 | CARSON VALLEY | 804 | yes |
| 6-61 | CAMEO AREA | NA | NA |
| 6-62 | RACE TRACK VALLEY | NA | NA |
| 6-63 | HIDDEN VALLEY | NA | NA |
| 6-64 | MARBLE CANYON AREA | NA | NA |
| 6-65 | COTTONWOOD SPRING AREA | NA | NA |
| 6-66 | LEE FLAT | 158 | yes |
| 6-67 | MARTIS VALLEY | 1416 | no |
| 6-68 | SANTA ROSA FLAT | NA | NA |
| 6-69 | KELSO LANDER VALLEY | 778 | yes |
| 6-7 | ANTELOPE VALLEY | 784 | yes |
| 6-70 | CACTUS FLAT | NA | NA |
| 6-71 | LOST LAKE VALLEY | NA | NA |
| 6-72 | COLES FLAT | NA | NA |
| 6-73 | WILD HORSE MESA AREA | NA | NA |
| 6-74 | HARRISBURG FLATS | NA | NA |
| 6-75 | WILDROSE CANYON | NA | NA |
| 6-76 | BROWN MOUNTAIN VALLEY | NA | NA |
| 6-77 | GRASS VALLEY | NA | NA |
| 6-78 | DENNING SPRING VALLEY | NA | NA |
| 6-79 | CALIFORNIA VALLEY | 850 | yes |
| 6-8 | BRIDGEPORT VALLEY | 563 | yes |
| 6-80 | MIDDLE PARK CANYON | NA | NA |
| 6-81 | BUTTE VALLEY | NA | NA |
| 6-82 | SPRING CANYON VALLEY | NA | NA |
| 6-84 | GREENWATER VALLEY | NA | NA |
| 6-85 | GOLD VALLEY | NA | NA |
| 6-86 | RHODES HILL AREA | NA | NA |
| 6-88 | OWL LAKE VALLEY | NA | NA |
| 6-89 | KANE WASH AREA | 650 | yes |
| 6-9 | MONO VALLEY | 773 | yes |
| 6-90 | CADY FAULT AREA | NA | NA |
| 6-91 | COW HEAD LAKE VALLEY | NA | NA |
| 6-92 | PINE CREEK VALLEY | NA | NA |
| 6-93 | HARVEY VALLEY | NA | NA |
| 6-94 | GRASSHOPPER VALLEY | 640 | yes |
| 6-95 | DRY VALLEY | 1171 | yes |
| 6-96 | EAGLE LAKE AREA | 410 | no |
| 6-97 | HORSE LAKE VALLEY | NA | NA |
| 6-98 | TULEDAD CANYON VALLEY | NA | NA |
| 6-99 | PAINTERS FLAT | NA | NA |
| 7-1 | LANFAIR VALLEY | 1301 | no |
| 7-10 | TWENTYNINE PALMS VALLEY | 790 | no |
| 7-11 | COPPER MOUNTAIN VALLEY | 834 | no |
| 7-12 | WARREN VALLEY | 979 | no |
| 7-13 | DEADMAN VALLEY | 1237 | no |
| 7-14 | LAVIC VALLEY | 624 | no |
| 7-15 | BESSEMER VALLEY | NA | NA |
| 7-16 | AMES VALLEY | 904 | yes |
| 7-17 | MEANS VALLEY | 1015 | yes |
| 7-18 | JOHNSON VALLEY | 959 | yes |
| 7-19 | LUCERNE VALLEY | 761 | yes |
| 7-2 | FENNER VALLEY | 1204 | yes |
| 7-20 | MORONGO VALLEY | 715 | yes |
| 7-21 | COACHELLA VALLEY | 1331 | no |
| 7-22 | WEST SALTON SEA | 373 | no |
| 7-24 | BORREGO VALLEY | 1100 | yes |
| 7-25 | OCOTILLO-CLARK VALLEY | 885 | yes |
| 7-26 | TERWILLIGER VALLEY | 1084 | yes |
| 7-27 | SAN FELIPE VALLEY | 745 | yes |
| 7-28 | VALLECITO-CARRIZO VALLEY | 1056 | yes |
| 7-29 | COYOTE WELLS VALLEY | 879 | yes |
| 7-3 | WARD VALLEY | 880 | yes |
| 7-30 | IMPERIAL VALLEY | 1233 | yes |
| 7-31 | OROCOPIA VALLEY | 1177 | no |
| 7-32 | CHOCOLATE VALLEY | 814 | yes |
| 7-33 | EAST SALTON SEA | 759 | yes |
| 7-34 | AMOS VALLEY | 705 | no |
| 7-35 | OGILBY VALLEY | 524 | yes |
| 7-36 | YUMA VALLEY | 461 | no |
| 7-37 | ARROYO SECO VALLEY | 625 | yes |
| 7-38 | PALO VERDE VALLEY | 593 | no |
| 7-39 | PALO VERDE MESA | 673 | no |
| 7-4 | RICE VALLEY | 510 | yes |
| 7-40 | QUIEN SABE POINT VALLEY | 492 | yes |
| 7-41 | CALZONA VALLEY | 419 | no |
| 7-42 | VIDAL VALLEY | 980 | yes |
| 7-43 | CHEMEHUEVI VALLEY | 598 | no |
| 7-44 | NEEDLES VALLEY | 469 | no |
| 7-45 | PIUTE VALLEY | 1260 | yes |
| 7-46 | CANEBRAKE VALLEY | 842 | yes |
| 7-47 | JACUMBA VALLEY | 815 | yes |
| 7-48 | HELENDALE FAULT VALLEY | 663 | yes |
| 7-49 | PIPES CANYON FAULT VALLEY | 1041 | yes |
| 7-5 | CHUCKWALLA VALLEY | 1202 | no |
| 7-50 | IRON RIDGE AREA | NA | NA |
| 7-51 | LOST HORSE VALLEY | 735 | yes |
| 7-52 | PLEASANT VALLEY | NA | NA |
| 7-53 | HEXIE MOUNTAIN AREA | NA | NA |
| 7-54 | BUCK RIDGE FAULT VALLEY | NA | NA |
| 7-55 | COLLINS VALLEY | 860 | yes |
| 7-56 | YAQUI WELL AREA | 526 | no |
| 7-59 | MASON VALLEY | 350 | yes |
| 7-6 | PINTO VALLEY | 853 | yes |
| 7-61 | DAVIES VALLEY | NA | NA |
| 7-62 | JOSHUA TREE | 804 | no |
| 7-63 | VANDEVENTER FLAT | 953 | yes |
| 7-7 | CADIZ VALLEY | 1015 | yes |
| 7-8 | BRISTOL VALLEY | 597 | no |
| 7-9 | DALE VALLEY | 719 | yes |
| 8-1 | COASTAL PLAIN OF ORANGE COUNTY | 1253 | no |
| 8-2 | UPPER SANTA ANA VALLEY | 1225 | no |
| 8-4 | ELSINORE | 1555 | no |
| 8-5 | SAN JACINTO | 1787 | yes |
| 8-6 | HEMET LAKE VALLEY | 1154 | yes |
| 8-7 | BIG MEADOWS VALLEY | 352 | no |
| 8-8 | SEVEN OAKS VALLEY | 756 | yes |
| 8-9 | BEAR VALLEY | 722 | no |
| 9-1 | SAN JUAN VALLEY | 455 | yes |
| 9-10 | SAN PASQUAL VALLEY | 1696 | yes |
| 9-11 | SANTA MARIA VALLEY | 1678 | yes |
| 9-12 | SAN DIEGUITO CREEK | 754 | yes |
| 9-13 | POWAY VALLEY | 1680 | yes |
| 9-14 | MISSION VALLEY | 459 | yes |
| 9-15 | SAN DIEGO RIVER VALLEY | 2059 | yes |
| 9-16 | EL CAJON VALLEY | 1533 | yes |
| 9-17 | SWEETWATER VALLEY | 544 | no |
| 9-18 | OTAY VALLEY | 638 | no |
| 9-19 | TIA JUANA | 375 | no |
| 9-2 | SAN MATEO VALLEY | NA | NA |
| 9-22 | BATIQUITOS LAGOON VALLEY | 1012 | yes |
| 9-23 | SAN ELIJO VALLEY | 1650 | yes |
| 9-24 | PAMO VALLEY | 1029 | yes |
| 9-25 | RANCHITA TOWN AREA | 1266 | yes |
| 9-27 | COTTONWOOD VALLEY | 1276 | yes |
| 9-28 | CAMPO VALLEY | 1654 | no |
| 9-29 | POTRERO VALLEY | 1442 | yes |
| 9-3 | SAN ONOFRE VALLEY | 525 | yes |
| 9-32 | SAN MARCOS AREA | 296 | no |
| 9-4 | SANTA MARGARITA VALLEY | 234 | yes |
| 9-5 | TEMECULA VALLEY | 1261 | no |
| 9-6 | CAHUILLA VALLEY | 1340 | yes |
| 9-7 | SAN LUIS REY VALLEY | 1607 | yes |
| 9-8 | WARNER VALLEY | 966 | no |
| 9-9 | ESCONDIDO VALLEY | 958 | yes |
| Hgb-1-1 | SMITH RIVER PLAIN HIGHLANDS | 361 | no |
| Hgb-1-10 | EEL RIVER VALLEY HIGHLANDS | 481 | yes |
| Hgb-1-11 | COVELO ROUND VALLEY HIGHLANDS | 461 | no |
| Hgb-1-12 | LAYTONVILLE VALLEY HIGHLANDS | 597 | no |
| Hgb-1-13 | LITTLE LAKE VALLEY HIGHLANDS | 548 | yes |
| Hgb-1-14 | LOWER KLAMATH RIVER VALLEY HIGHLANDS | 478 | yes |
| Hgb-1-15 | HAPPY CAMP TOWN AREA HIGHLANDS | 487 | yes |
| Hgb-1-16 | SEIAD VALLEY HIGHLANDS | 746 | yes |
| Hgb-1-17 | BRAY TOWN AREA HIGHLANDS | 833 | yes |
| Hgb-1-18 | RED ROCK VALLEY HIGHLANDS | 664 | yes |
| Hgb-1-19 | ANDERSON VALLEY HIGHLANDS | 516 | no |
| Hgb-1-2 | KLAMATH RIVER VALLEY HIGHLANDS | 734 | no |
| Hgb-1-20 | GARCIA RIVER VALLEY HIGHLANDS | 426 | yes |
| Hgb-1-21 | FORT BRAGG TERRACE AREA HIGHLANDS | 523 | yes |
| Hgb-1-22 | FAIRCHILD SWAMP VALLEY HIGHLANDS | NA | NA |
| Hgb-1-25 | PRAIRIE CREEK AREA HIGHLANDS | 195 | no |
| Hgb-1-26 | REDWOOD CREEK AREA HIGHLANDS | 417 | no |
| Hgb-1-27 | BIG LAGOON AREA HIGHLANDS | 380 | yes |
| Hgb-1-28 | MATTOLE RIVER VALLEY HIGHLANDS | 424 | no |
| Hgb-1-29 | HONEYDEW TOWN AREA HIGHLANDS | 498 | yes |
| Hgb-1-3 | BUTTE VALLEY HIGHLANDS | 870 | no |
| Hgb-1-30 | PEPPERWOOD TOWN AREA HIGHLANDS | 432 | no |
| Hgb-1-31 | WEOTT TOWN AREA HIGHLANDS | 470 | yes |
| Hgb-1-32 | GARBERVILLE TOWN AREA HIGHLANDS | 498 | no |
| Hgb-1-33 | LARABEE VALLEY HIGHLANDS | 449 | no |
| Hgb-1-34 | DINSMORES TOWN AREA HIGHLANDS | 470 | yes |
| Hgb-1-35 | HYAMPOM VALLEY HIGHLANDS | 654 | yes |
| Hgb-1-36 | HETTENSHAW VALLEY HIGHLANDS | 395 | yes |
| Hgb-1-37 | COTTONEVA CREEK VALLEY HIGHLANDS | 554 | yes |
| Hgb-1-38 | LOWER LAYTONVILLE VALLEY HIGHLANDS | 591 | no |
| Hgb-1-39 | BRANSCOMB TOWN AREA HIGHLANDS | 472 | no |
| Hgb-1-4 | SHASTA VALLEY HIGHLANDS | 722 | yes |
| Hgb-1-40 | TEN MILE RIVER VALLEY HIGHLANDS | 484 | no |
| Hgb-1-41 | LITTLE VALLEY HIGHLANDS | 386 | yes |
| Hgb-1-42 | SHERWOOD VALLEY HIGHLANDS | 433 | no |
| Hgb-1-43 | WILLIAMS VALLEY HIGHLANDS | 515 | yes |
| Hgb-1-44 | EDEN VALLEY HIGHLANDS | 650 | yes |
| Hgb-1-45 | BIG RIVER VALLEY HIGHLANDS | 477 | yes |
| Hgb-1-46 | NAVARRO RIVER VALLEY HIGHLANDS | 486 | yes |
| Hgb-1-48 | GRAVELLY VALLEY HIGHLANDS | 643 | yes |
| Hgb-1-49 | ANNAPOLIS OHLSON RANCH FM HIGHLANDS | 499 | yes |
| Hgb-1-5 | SCOTT RIVER VALLEY HIGHLANDS | 522 | yes |
| Hgb-1-50 | KNIGHTS VALLEY HIGHLANDS | 793 | yes |
| Hgb-1-51 | POTTER VALLEY HIGHLANDS | 539 | yes |
| Hgb-1-52 | UKIAH VALLEY HIGHLANDS | 614 | no |
| Hgb-1-53 | SANEL VALLEY HIGHLANDS | 525 | no |
| Hgb-1-54 | ALEXANDER VALLEY HIGHLANDS | 1022 | yes |
| Hgb-1-55 | SANTA ROSA VALLEY HIGHLANDS | 1112 | yes |
| Hgb-1-56 | McDOWELL VALLEY HIGHLANDS | 540 | no |
| Hgb-1-57 | BODEGA BAY AREA HIGHLANDS | 667 | yes |
| Hgb-1-59 | WILSON GROVE FORMATION HIGHLANDS | 755 | yes |
| Hgb-1-6 | HAYFORK VALLEY HIGHLANDS | 669 | yes |
| Hgb-1-60 | LOWER RUSSIAN RIVER VALLEY HIGHLANDS | 615 | yes |
| Hgb-1-61 | FORT ROSS TERRACE DEPOSITS HIGHLANDS | 551 | no |
| Hgb-1-62 | WILSON POINT AREA HIGHLANDS | 505 | yes |
| Hgb-1-7 | HOOPA VALLEY HIGHLANDS | 552 | yes |
| Hgb-1-8 | MAD RIVER VALLEY HIGHLANDS | 463 | yes |
| Hgb-1-9 | EUREKA PLAIN HIGHLANDS | 462 | no |
| Hgb-2-1 | PETALUMA VALLEY HIGHLANDS | 1082 | yes |
| Hgb-2-10 | LIVERMORE VALLEY HIGHLANDS | 740 | yes |
| Hgb-2-11 | SUNOL VALLEY HIGHLANDS | 731 | no |
| Hgb-2-19 | KENWOOD VALLEY HIGHLANDS | 1281 | yes |
| Hgb-2-2 | NAPA-SONOMA VALLEY HIGHLANDS | 1247 | yes |
| Hgb-2-22 | HALF MOON BAY TERRACE HIGHLANDS | 892 | yes |
| Hgb-2-24 | SAN GREGORIO VALLEY HIGHLANDS | 954 | yes |
| Hgb-2-26 | PESCADERO VALLEY HIGHLANDS | 826 | yes |
| Hgb-2-27 | SAND POINT AREA HIGHLANDS | 758 | yes |
| Hgb-2-28 | ROSS VALLEY HIGHLANDS | 653 | yes |
| Hgb-2-29 | SAN RAFAEL VALLEY HIGHLANDS | 759 | yes |
| Hgb-2-3 | SUISUN-FAIRFIELD VALLEY HIGHLANDS | 1071 | yes |
| Hgb-2-30 | NOVATO VALLEY HIGHLANDS | 833 | no |
| Hgb-2-31 | ARROYO DEL HAMBRE VALLEY HIGHLANDS | 845 | yes |
| Hgb-2-35 | WESTSIDE HIGHLANDS | 689 | no |
| Hgb-2-36 | SAN PEDRO VALLEY HIGHLANDS | 515 | yes |
| Hgb-2-4 | PITTSBURG PLAIN HIGHLANDS | 828 | no |
| Hgb-2-5 | CLAYTON VALLEY HIGHLANDS | 925 | yes |
| Hgb-2-6 | YGNACIO VALLEY HIGHLANDS | 742 | yes |
| Hgb-2-7 | SAN RAMON VALLEY HIGHLANDS | 768 | yes |
| Hgb-2-8 | CASTRO VALLEY HIGHLANDS | 666 | yes |
| Hgb-2-9 | SANTA CLARA VALLEY HIGHLANDS | 948 | yes |
| Hgb-3-12 | SANTA MARIA HIGHLANDS | 921 | yes |
| Hgb-3-13 | CUYAMA VALLEY HIGHLANDS | 852 | no |
| Hgb-3-14 | SAN ANTONIO CREEK VALLEY HIGHLANDS | 1450 | yes |
| Hgb-3-15 | SANTA YNEZ RIVER VALLEY HIGHLANDS | 1089 | yes |
| Hgb-3-16 | GOLETA HIGHLANDS | 1577 | yes |
| Hgb-3-17 | SANTA BARBARA HIGHLANDS | 1278 | yes |
| Hgb-3-18 | CARPINTERIA HIGHLANDS | 1575 | yes |
| Hgb-3-19 | CARRIZO PLAIN HIGHLANDS | 599 | yes |
| Hgb-3-2 | PAJARO VALLEY HIGHLANDS | 1215 | yes |
| Hgb-3-20 | ANO NUEVO AREA HIGHLANDS | 501 | no |
| Hgb-3-21 | SANTA CRUZ PURISIMA FORMATION HIGHLANDS | 1170 | yes |
| Hgb-3-22 | SANTA ANA VALLEY HIGHLANDS | 1071 | yes |
| Hgb-3-23 | UPPER SANTA ANA VALLEY HIGHLANDS | 128 | yes |
| Hgb-3-24 | QUIEN SABE VALLEY HIGHLANDS | NA | NA |
| Hgb-3-25 | TRES PINOS VALLEY HIGHLANDS | 678 | yes |
| Hgb-3-26 | WEST SANTA CRUZ TERRACE HIGHLANDS | 1307 | yes |
| Hgb-3-27 | SCOTTS VALLEY HIGHLANDS | 1271 | yes |
| Hgb-3-28 | SAN BENITO RIVER VALLEY HIGHLANDS | 642 | no |
| Hgb-3-29 | DRY LAKE VALLEY HIGHLANDS | 728 | yes |
| Hgb-3-3 | GILROY-HOLLISTER VALLEY HIGHLANDS | 964 | yes |
| Hgb-3-30 | BITTER WATER VALLEY HIGHLANDS | 749 | yes |
| Hgb-3-31 | HERNANDEZ VALLEY HIGHLANDS | 340 | yes |
| Hgb-3-32 | PEACH TREE VALLEY HIGHLANDS | 517 | yes |
| Hgb-3-33 | SAN CARPOFORO VALLEY HIGHLANDS | 991 | yes |
| Hgb-3-34 | ARROYO DE LA CRUZ VALLEY HIGHLANDS | 176 | yes |
| Hgb-3-35 | SAN SIMEON VALLEY HIGHLANDS | 693 | yes |
| Hgb-3-36 | SANTA ROSA VALLEY HIGHLANDS | 512 | yes |
| Hgb-3-37 | VILLA VALLEY HIGHLANDS | 583 | yes |
| Hgb-3-38 | CAYUCOS VALLEY HIGHLANDS | 670 | yes |
| Hgb-3-39 | OLD VALLEY HIGHLANDS | 670 | yes |
| Hgb-3-4.1 | 180/400 FOOT AQUIFER; EAST SIDE AQUIFER; SEASIDE AREA; LANGLEY AREA; CORRAL DE TIERRA AREA HIGHLANDS | 1252 | no |
| Hgb-3-4.2 | FOREBAY AQUIFER; UPPER VALLEY AQUIFER HIGHLANDS | 828 | yes |
| Hgb-3-4.06 | PASO ROBLES AREA HIGHLANDS | 877 | yes |
| Hgb-3-40 | TORO VALLEY HIGHLANDS | 675 | yes |
| Hgb-3-41 | MORRO VALLEY HIGHLANDS | 596 | yes |
| Hgb-3-42 | CHORRO VALLEY HIGHLANDS | 1164 | yes |
| Hgb-3-43 | RINCONADA VALLEY HIGHLANDS | 680 | yes |
| Hgb-3-44 | POZO VALLEY HIGHLANDS | 1038 | yes |
| Hgb-3-45 | HUASNA VALLEY HIGHLANDS | 580 | yes |
| Hgb-3-46 | RAFAEL VALLEY HIGHLANDS | NA | NA |
| Hgb-3-47 | BIG SPRING AREA HIGHLANDS | 605 | yes |
| Hgb-3-49 | MONTECITO HIGHLANDS | 1402 | yes |
| Hgb-3-5 | CHOLAME VALLEY HIGHLANDS | 513 | yes |
| Hgb-3-50 | FELTON AREA HIGHLANDS | 997 | yes |
| Hgb-3-51 | MAJORS CREEK HIGHLANDS | 869 | yes |
| Hgb-3-52 | NEEDLE ROCK POINT HIGHLANDS | 758 | yes |
| Hgb-3-53 | FOOTHILL HIGHLANDS | 1213 | no |
| Hgb-3-6 | LOCKWOOD VALLEY HIGHLANDS | 842 | yes |
| Hgb-3-7 | CARMEL VALLEY HIGHLANDS | 1344 | no |
| Hgb-3-8 | LOS OSOS VALLEY HIGHLANDS | 583 | yes |
| Hgb-3-9 | SAN LUIS OBISPO VALLEY HIGHLANDS | 830 | yes |
| Hgb-4-1 | UPPER OJAI VALLEY HIGHLANDS | 979 | yes |
| Hgb-4-10 | CONEJO HIGHLANDS | 1457 | yes |
| Hgb-4-11 | COASTAL PLAIN OF LOS ANGELES HIGHLANDS | 1123 | yes |
| Hgb-4-12 | SAN FERNANDO VALLEY HIGHLANDS | 955 | no |
| Hgb-4-13 | SAN GABRIEL VALLEY HIGHLANDS | 800 | no |
| Hgb-4-15 | TIERRA REJADA HIGHLANDS | 1044 | yes |
| Hgb-4-16 | HIDDEN VALLEY HIGHLANDS | 1319 | yes |
| Hgb-4-17 | LOCKWOOD VALLEY HIGHLANDS | 791 | no |
| Hgb-4-18 | HUNGRY VALLEY HIGHLANDS | 455 | no |
| Hgb-4-19 | THOUSAND OAKS AREA HIGHLANDS | 1018 | yes |
| Hgb-4-2 | OJAI VALLEY HIGHLANDS | 1171 | yes |
| Hgb-4-20 | RUSSELL VALLEY HIGHLANDS | 671 | yes |
| Hgb-4-22 | MALIBU VALLEY HIGHLANDS | 1155 | yes |
| Hgb-4-23 | RAYMOND HIGHLANDS | 904 | no |
| Hgb-4-3 | VENTURA RIVER VALLEY HIGHLANDS | 1191 | yes |
| Hgb-4-4 | SANTA CLARA RIVER VALLEY HIGHLANDS | 1141 | yes |
| Hgb-4-5 | ACTON VALLEY HIGHLANDS | 1196 | yes |
| Hgb-4-6 | PLEASANT VALLEY HIGHLANDS | 1194 | yes |
| Hgb-4-7 | ARROYO SANTA ROSA VALLEY HIGHLANDS | 1857 | yes |
| Hgb-4-8 | LAS POSAS VALLEY HIGHLANDS | 1577 | yes |
| Hgb-4-9 | SIMI VALLEY HIGHLANDS | 891 | no |
| Hgb-5-1 | GOOSE LAKE HIGHLANDS | 685 | yes |
| Hgb-5-10 | AMERICAN VALLEY HIGHLANDS | 666 | no |
| Hgb-5-11 | MOHAWK VALLEY HIGHLANDS | 979 | no |
| Hgb-5-12 | SIERRA VALLEY HIGHLANDS | 763 | no |
| Hgb-5-13 | UPPER LAKE VALLEY HIGHLANDS | 507 | yes |
| Hgb-5-14 | SCOTTS VALLEY HIGHLANDS | 508 | yes |
| Hgb-5-15 | BIG VALLEY HIGHLANDS | 887 | yes |
| Hgb-5-16 | HIGH VALLEY HIGHLANDS | 586 | yes |
| Hgb-5-17 | BURNS VALLEY HIGHLANDS | 550 | no |
| Hgb-5-18 | COYOTE VALLEY HIGHLANDS | 745 | yes |
| Hgb-5-19 | COLLAYOMI VALLEY HIGHLANDS | 652 | yes |
| Hgb-5-2 | ALTURAS AREA HIGHLANDS | 1021 | yes |
| Hgb-5-20 | BERRYESSA VALLEY HIGHLANDS | 451 | yes |
| Hgb-5-21.50 | RED BLUFF HIGHLANDS | 555 | no |
| Hgb-5-21.51 | CORNING HIGHLANDS | 597 | no |
| Hgb-5-21.52 | COLUSA HIGHLANDS | 481 | no |
| Hgb-5-21.53 | BEND HIGHLANDS | 651 | no |
| Hgb-5-21.54 | ANTELOPE HIGHLANDS | 484 | no |
| Hgb-5-21.55 | DYE CREEK HIGHLANDS | 499 | no |
| Hgb-5-21.56 | LOS MOLINOS HIGHLANDS | 712 | no |
| Hgb-5-21.57 | VINA HIGHLANDS | 1424 | yes |
| Hgb-5-21.58 | WEST BUTTE HIGHLANDS | 1246 | yes |
| Hgb-5-21.59 | EAST BUTTE HIGHLANDS | 1046 | yes |
| Hgb-5-21.60 | NORTH YUBA HIGHLANDS | 1173 | yes |
| Hgb-5-21.61 | SOUTH YUBA HIGHLANDS | 1262 | yes |
| Hgb-5-21.62 | SUTTER HIGHLANDS | 1053 | yes |
| Hgb-5-21.64 | NORTH AMERICAN HIGHLANDS | 1383 | yes |
| Hgb-5-21.65 | SOUTH AMERICAN HIGHLANDS | 1376 | yes |
| Hgb-5-21.66 | SOLANO HIGHLANDS | 753 | no |
| Hgb-5-21.67 | YOLO HIGHLANDS | 429 | yes |
| Hgb-5-21.68 | CAPAY VALLEY HIGHLANDS | 604 | no |
| Hgb-5-22.01 | EASTERN SAN JOAQUIN HIGHLANDS | 1211 | yes |
| Hgb-5-22.02 | MODESTO HIGHLANDS | 1466 | yes |
| Hgb-5-22.03 | TURLOCK HIGHLANDS | 1521 | yes |
| Hgb-5-22.04 | MERCED HIGHLANDS | 1318 | no |
| Hgb-5-22.06 | MADERA HIGHLANDS | 1695 | yes |
| Hgb-5-22.07 | DELTA-MENDOTA HIGHLANDS | 722 | no |
| Hgb-5-22.08 | KINGS HIGHLANDS | 1563 | yes |
| Hgb-5-22.09 | WESTSIDE HIGHLANDS | 394 | yes |
| Hgb-5-22.10 | PLEASANT VALLEY HIGHLANDS | 955 | yes |
| Hgb-5-22.11 | KAWEAH HIGHLANDS | 1101 | yes |
| Hgb-5-22.13 | TULE HIGHLANDS | 971 | yes |
| Hgb-5-22.14 | KERN COUNTY HIGHLANDS | 1079 | yes |
| Hgb-5-22.15 | TRACY HIGHLANDS | 736 | yes |
| Hgb-5-22.16 | COSUMNES HIGHLANDS | 1299 | yes |
| Hgb-5-23 | PANOCHE VALLEY HIGHLANDS | 828 | yes |
| Hgb-5-25 | KERN RIVER VALLEY HIGHLANDS | 855 | yes |
| Hgb-5-26 | WALKER BASIN CREEK VALLEY HIGHLANDS | 809 | yes |
| Hgb-5-27 | CUMMINGS VALLEY HIGHLANDS | 1020 | no |
| Hgb-5-28 | TEHACHAPI VALLEY WEST HIGHLANDS | 1265 | yes |
| Hgb-5-29 | CASTAC LAKE VALLEY HIGHLANDS | 964 | yes |
| Hgb-5-3 | JESS VALLEY HIGHLANDS | 332 | no |
| Hgb-5-30 | LOWER LAKE VALLEY HIGHLANDS | 815 | yes |
| Hgb-5-31 | LONG VALLEY HIGHLANDS | 412 | no |
| Hgb-5-35 | MCCLOUD AREA HIGHLANDS | 705 | yes |
| Hgb-5-36 | ROUND VALLEY HIGHLANDS | 845 | no |
| Hgb-5-37 | TOAD WELL AREA HIGHLANDS | NA | NA |
| Hgb-5-38 | PONDOSA TOWN AREA HIGHLANDS | NA | NA |
| Hgb-5-4 | BIG VALLEY HIGHLANDS | 936 | yes |
| Hgb-5-40 | HOT SPRINGS VALLEY HIGHLANDS | 708 | yes |
| Hgb-5-41 | EGG LAKE VALLEY HIGHLANDS | 685 | yes |
| Hgb-5-43 | ROCK PRAIRIE VALLEY HIGHLANDS | 1635 | yes |
| Hgb-5-44 | LONG VALLEY HIGHLANDS | NA | NA |
| Hgb-5-45 | CAYTON VALLEY HIGHLANDS | 715 | yes |
| Hgb-5-46 | LAKE BRITTON AREA HIGHLANDS | 672 | no |
| Hgb-5-47 | GOOSE VALLEY HIGHLANDS | 548 | yes |
| Hgb-5-48 | BURNEY CREEK VALLEY HIGHLANDS | 621 | yes |
| Hgb-5-49 | DRY BURNEY CREEK VALLEY HIGHLANDS | 755 | yes |
| Hgb-5-5 | FALL RIVER VALLEY HIGHLANDS | 869 | yes |
| Hgb-5-50 | NORTH FORK BATTLE CREEK HIGHLANDS | 868 | yes |
| Hgb-5-51 | BUTTE CREEK VALLEY HIGHLANDS | 590 | yes |
| Hgb-5-52 | GRAYS VALLEY HIGHLANDS | 514 | yes |
| Hgb-5-53 | DIXIE VALLEY HIGHLANDS | 746 | yes |
| Hgb-5-54 | ASH VALLEY HIGHLANDS | 1811 | yes |
| Hgb-5-56 | YELLOW CREEK VALLEY HIGHLANDS | 623 | yes |
| Hgb-5-57 | LAST CHANCE CREEK VALLEY HIGHLANDS | 626 | yes |
| Hgb-5-58 | CLOVER VALLEY HIGHLANDS | 538 | yes |
| Hgb-5-59 | GRIZZLY VALLEY HIGHLANDS | 647 | yes |
| Hgb-5-6 | REDDING AREA HIGHLANDS | 689 | yes |
| Hgb-5-60 | HUMBUG VALLEY HIGHLANDS | 984 | no |
| Hgb-5-61 | CHROME TOWN AREA HIGHLANDS | 479 | yes |
| Hgb-5-62 | ELK CREEK AREA HIGHLANDS | 356 | yes |
| Hgb-5-63 | STONYFORD TOWN AREA HIGHLANDS | 264 | no |
| Hgb-5-64 | BEAR VALLEY HIGHLANDS | 318 | yes |
| Hgb-5-65 | LITTLE INDIAN VALLEY HIGHLANDS | 463 | yes |
| Hgb-5-66 | CLEAR LAKE CACHE FORMATION HIGHLANDS | 566 | yes |
| Hgb-5-68 | POPE VALLEY HIGHLANDS | 949 | yes |
| Hgb-5-69 | YOSEMITE VALLEY HIGHLANDS | 1630 | yes |
| Hgb-5-7 | LAKE ALMANOR VALLEY HIGHLANDS | 659 | no |
| Hgb-5-70 | LOS BANOS CREEK VALLEY HIGHLANDS | 815 | yes |
| Hgb-5-71 | VALLECITOS CREEK VALLEY HIGHLANDS | 660 | yes |
| Hgb-5-8 | MOUNTAIN MEADOWS VALLEY HIGHLANDS | 775 | yes |
| Hgb-5-82 | CUDDY CANYON VALLEY HIGHLANDS | 809 | yes |
| Hgb-5-83 | CUDDY RANCH AREA HIGHLANDS | 1072 | yes |
| Hgb-5-84 | CUDDY VALLEY HIGHLANDS | 994 | yes |
| Hgb-5-85 | MIL POTRERO AREA HIGHLANDS | 725 | no |
| Hgb-5-86 | JOSEPH CREEK HIGHLANDS | 905 | yes |
| Hgb-5-87 | MIDDLE FORK FEATHER RIVER HIGHLANDS | 889 | yes |
| Hgb-5-88 | STONY GORGE RESERVOIR HIGHLANDS | 452 | no |
| Hgb-5-89 | SQUAW FLAT HIGHLANDS | 642 | yes |
| Hgb-5-9 | INDIAN VALLEY HIGHLANDS | 632 | no |
| Hgb-5-90 | FUNKS CREEK HIGHLANDS | 316 | yes |
| Hgb-5-91 | ANTELOPE CREEK HIGHLANDS | 227 | yes |
| Hgb-5-92 | BLANCHARD VALLEY HIGHLANDS | 300 | yes |
| Hgb-5-93 | NORTH FORK CACHE CREEK HIGHLANDS | 325 | no |
| Hgb-5-94 | MIDDLE CREEK HIGHLANDS | 523 | yes |
| Hgb-5-95 | MEADOW VALLEY HIGHLANDS | 441 | no |
| Hgb-6-1 | SURPRISE VALLEY HIGHLANDS | 851 | yes |
| Hgb-6-10 | ADOBE LAKE VALLEY HIGHLANDS | 448 | no |
| Hgb-6-100 | SECRET VALLEY HIGHLANDS | 1003 | yes |
| Hgb-6-101 | BULL FLAT HIGHLANDS | NA | NA |
| Hgb-6-104 | LONG VALLEY HIGHLANDS | 655 | yes |
| Hgb-6-105 | SLINKARD VALLEY HIGHLANDS | 550 | yes |
| Hgb-6-106 | LITTLE ANTELOPE VALLEY HIGHLANDS | 1090 | yes |
| Hgb-6-107 | SWEETWATER FLAT HIGHLANDS | NA | NA |
| Hgb-6-11 | LONG VALLEY HIGHLANDS | 754 | no |
| Hgb-6-12 | OWENS VALLEY HIGHLANDS | 893 | no |
| Hgb-6-13 | BLACK SPRINGS VALLEY HIGHLANDS | NA | NA |
| Hgb-6-14 | FISH LAKE VALLEY HIGHLANDS | 605 | yes |
| Hgb-6-15 | DEEP SPRINGS VALLEY HIGHLANDS | 469 | no |
| Hgb-6-16 | EUREKA VALLEY HIGHLANDS | 754 | yes |
| Hgb-6-17 | SALINE VALLEY HIGHLANDS | 688 | yes |
| Hgb-6-18 | DEATH VALLEY HIGHLANDS | NA | NA |
| Hgb-6-19 | WINGATE VALLEY HIGHLANDS | NA | NA |
| Hgb-6-2 | MADELINE PLAINS HIGHLANDS | 944 | yes |
| Hgb-6-20 | MIDDLE AMARGOSA VALLEY HIGHLANDS | 819 | yes |
| Hgb-6-21 | LOWER KINGSTON VALLEY HIGHLANDS | NA | NA |
| Hgb-6-22 | UPPER KINGSTON VALLEY HIGHLANDS | 550 | yes |
| Hgb-6-23 | RIGGS VALLEY HIGHLANDS | 530 | no |
| Hgb-6-24 | RED PASS VALLEY HIGHLANDS | 730 | no |
| Hgb-6-25 | BICYCLE VALLEY HIGHLANDS | NA | NA |
| Hgb-6-26 | AVAWATZ VALLEY HIGHLANDS | NA | NA |
| Hgb-6-27 | LEACH VALLEY HIGHLANDS | NA | NA |
| Hgb-6-28 | PAHRUMP VALLEY HIGHLANDS | NA | NA |
| Hgb-6-29 | MESQUITE VALLEY HIGHLANDS | NA | NA |
| Hgb-6-3 | WILLOW CREEK VALLEY HIGHLANDS | 620 | yes |
| Hgb-6-30 | IVANPAH VALLEY HIGHLANDS | 1250 | yes |
| Hgb-6-31 | KELSO VALLEY HIGHLANDS | 894 | yes |
| Hgb-6-32 | BROADWELL VALLEY HIGHLANDS | NA | NA |
| Hgb-6-33 | SODA LAKE VALLEY HIGHLANDS | 564 | yes |
| Hgb-6-34 | SILVER LAKE VALLEY HIGHLANDS | 530 | no |
| Hgb-6-35 | CRONISE VALLEY HIGHLANDS | 610 | no |
| Hgb-6-36 | LANGFORD VALLEY HIGHLANDS | NA | NA |
| Hgb-6-37 | COYOTE LAKE VALLEY HIGHLANDS | 850 | yes |
| Hgb-6-38 | CAVES CANYON VALLEY HIGHLANDS | 783 | yes |
| Hgb-6-4 | HONEY LAKE VALLEY HIGHLANDS | 843 | no |
| Hgb-6-40 | LOWER MOJAVE RIVER VALLEY HIGHLANDS | 629 | no |
| Hgb-6-41 | MIDDLE MOJAVE RIVER VALLEY HIGHLANDS | 899 | yes |
| Hgb-6-42 | UPPER MOJAVE RIVER VALLEY HIGHLANDS | 958 | yes |
| Hgb-6-43 | EL MIRAGE VALLEY HIGHLANDS | 1018 | no |
| Hgb-6-44 | ANTELOPE VALLEY HIGHLANDS | 1039 | yes |
| Hgb-6-45 | TEHACHAPI VALLEY EAST HIGHLANDS | 890 | yes |
| Hgb-6-46 | FREMONT VALLEY HIGHLANDS | 789 | yes |
| Hgb-6-47 | HARPER VALLEY HIGHLANDS | 794 | yes |
| Hgb-6-48 | GOLDSTONE VALLEY HIGHLANDS | 980 | yes |
| Hgb-6-49 | SUPERIOR VALLEY HIGHLANDS | 678 | yes |
| Hgb-6-5 | TAHOE VALLEY HIGHLANDS | 607 | no |
| Hgb-6-50 | CUDDEBACK VALLEY HIGHLANDS | 665 | yes |
| Hgb-6-51 | PILOT KNOB VALLEY HIGHLANDS | NA | NA |
| Hgb-6-52 | SEARLES VALLEY HIGHLANDS | 668 | yes |
| Hgb-6-53 | SALT WELLS VALLEY HIGHLANDS | NA | NA |
| Hgb-6-54 | INDIAN WELLS VALLEY HIGHLANDS | 787 | yes |
| Hgb-6-55 | COSO VALLEY HIGHLANDS | NA | NA |
| Hgb-6-56 | ROSE VALLEY HIGHLANDS | 704 | yes |
| Hgb-6-57 | DARWIN VALLEY HIGHLANDS | NA | NA |
| Hgb-6-58 | PANAMINT VALLEY HIGHLANDS | NA | NA |
| Hgb-6-6 | CARSON VALLEY HIGHLANDS | 851 | yes |
| Hgb-6-61 | CAMEO AREA HIGHLANDS | NA | NA |
| Hgb-6-62 | RACE TRACK VALLEY HIGHLANDS | 756 | yes |
| Hgb-6-63 | HIDDEN VALLEY HIGHLANDS | NA | NA |
| Hgb-6-64 | MARBLE CANYON AREA HIGHLANDS | NA | NA |
| Hgb-6-65 | COTTONWOOD SPRING AREA HIGHLANDS | NA | NA |
| Hgb-6-66 | LEE FLAT HIGHLANDS | 158 | yes |
| Hgb-6-67 | MARTIS VALLEY HIGHLANDS | 1191 | yes |
| Hgb-6-68 | SANTA ROSA FLAT HIGHLANDS | NA | NA |
| Hgb-6-69 | KELSO LANDER VALLEY HIGHLANDS | 856 | yes |
| Hgb-6-7 | ANTELOPE VALLEY HIGHLANDS | 896 | yes |
| Hgb-6-70 | CACTUS FLAT HIGHLANDS | NA | NA |
| Hgb-6-71 | LOST LAKE VALLEY HIGHLANDS | NA | NA |
| Hgb-6-72 | COLES FLAT HIGHLANDS | NA | NA |
| Hgb-6-73 | WILD HORSE MESA AREA HIGHLANDS | NA | NA |
| Hgb-6-74 | HARRISBURG FLATS HIGHLANDS | NA | NA |
| Hgb-6-75 | WILDROSE CANYON HIGHLANDS | NA | NA |
| Hgb-6-76 | BROWN MOUNTAIN VALLEY HIGHLANDS | NA | NA |
| Hgb-6-77 | GRASS VALLEY HIGHLANDS | NA | NA |
| Hgb-6-78 | DENNING SPRING VALLEY HIGHLANDS | NA | NA |
| Hgb-6-79 | CALIFORNIA VALLEY HIGHLANDS | 850 | yes |
| Hgb-6-8 | BRIDGEPORT VALLEY HIGHLANDS | 528 | yes |
| Hgb-6-80 | MIDDLE PARK CANYON HIGHLANDS | NA | NA |
| Hgb-6-81 | BUTTE VALLEY HIGHLANDS | NA | NA |
| Hgb-6-82 | SPRING CANYON VALLEY HIGHLANDS | NA | NA |
| Hgb-6-84 | GREENWATER VALLEY HIGHLANDS | NA | NA |
| Hgb-6-85 | GOLD VALLEY HIGHLANDS | NA | NA |
| Hgb-6-86 | RHODES HILL AREA HIGHLANDS | NA | NA |
| Hgb-6-88 | OWL LAKE VALLEY HIGHLANDS | NA | NA |
| Hgb-6-89 | KANE WASH AREA HIGHLANDS | NA | NA |
| Hgb-6-9 | MONO VALLEY HIGHLANDS | 761 | yes |
| Hgb-6-90 | CADY FAULT AREA HIGHLANDS | NA | NA |
| Hgb-6-91 | COW HEAD LAKE VALLEY HIGHLANDS | NA | NA |
| Hgb-6-92 | PINE CREEK VALLEY HIGHLANDS | 514 | yes |
| Hgb-6-93 | HARVEY VALLEY HIGHLANDS | NA | NA |
| Hgb-6-94 | GRASSHOPPER VALLEY HIGHLANDS | 768 | yes |
| Hgb-6-95 | DRY VALLEY HIGHLANDS | 1154 | yes |
| Hgb-6-96 | EAGLE LAKE AREA HIGHLANDS | 389 | no |
| Hgb-6-97 | HORSE LAKE VALLEY HIGHLANDS | 528 | yes |
| Hgb-6-98 | TULEDAD CANYON VALLEY HIGHLANDS | NA | NA |
| Hgb-6-99 | PAINTERS FLAT HIGHLANDS | NA | NA |
| Hgb-7-1 | LANFAIR VALLEY HIGHLANDS | 1491 | yes |
| Hgb-7-10 | TWENTYNINE PALMS VALLEY HIGHLANDS | 848 | no |
| Hgb-7-11 | COPPER MOUNTAIN VALLEY HIGHLANDS | 869 | no |
| Hgb-7-12 | WARREN VALLEY HIGHLANDS | 895 | no |
| Hgb-7-13 | DEADMAN VALLEY HIGHLANDS | 556 | yes |
| Hgb-7-14 | LAVIC VALLEY HIGHLANDS | 624 | no |
| Hgb-7-15 | BESSEMER VALLEY HIGHLANDS | 670 | yes |
| Hgb-7-16 | AMES VALLEY HIGHLANDS | 776 | yes |
| Hgb-7-17 | MEANS VALLEY HIGHLANDS | 1006 | yes |
| Hgb-7-18 | JOHNSON VALLEY HIGHLANDS | 959 | yes |
| Hgb-7-19 | LUCERNE VALLEY HIGHLANDS | 828 | yes |
| Hgb-7-2 | FENNER VALLEY HIGHLANDS | 852 | yes |
| Hgb-7-20 | MORONGO VALLEY HIGHLANDS | 725 | yes |
| Hgb-7-21 | COACHELLA VALLEY HIGHLANDS | 1136 | no |
| Hgb-7-22 | WEST SALTON SEA HIGHLANDS | 1676 | yes |
| Hgb-7-24 | BORREGO VALLEY HIGHLANDS | 759 | no |
| Hgb-7-25 | OCOTILLO-CLARK VALLEY HIGHLANDS | 652 | yes |
| Hgb-7-26 | TERWILLIGER VALLEY HIGHLANDS | 1326 | yes |
| Hgb-7-27 | SAN FELIPE VALLEY HIGHLANDS | 1293 | yes |
| Hgb-7-28 | VALLECITO-CARRIZO VALLEY HIGHLANDS | 1145 | no |
| Hgb-7-29 | COYOTE WELLS VALLEY HIGHLANDS | 1228 | yes |
| Hgb-7-3 | WARD VALLEY HIGHLANDS | NA | NA |
| Hgb-7-30 | IMPERIAL VALLEY HIGHLANDS | NA | NA |
| Hgb-7-31 | OROCOPIA VALLEY HIGHLANDS | 650 | yes |
| Hgb-7-32 | CHOCOLATE VALLEY HIGHLANDS | 627 | yes |
| Hgb-7-33 | EAST SALTON SEA HIGHLANDS | 697 | yes |
| Hgb-7-34 | AMOS VALLEY HIGHLANDS | NA | NA |
| Hgb-7-35 | OGILBY VALLEY HIGHLANDS | 490 | yes |
| Hgb-7-36 | YUMA VALLEY HIGHLANDS | 293 | yes |
| Hgb-7-37 | ARROYO SECO VALLEY HIGHLANDS | NA | NA |
| Hgb-7-38 | PALO VERDE VALLEY HIGHLANDS | 484 | yes |
| Hgb-7-39 | PALO VERDE MESA HIGHLANDS | NA | NA |
| Hgb-7-4 | RICE VALLEY HIGHLANDS | NA | NA |
| Hgb-7-40 | QUIEN SABE POINT VALLEY HIGHLANDS | 484 | yes |
| Hgb-7-41 | CALZONA VALLEY HIGHLANDS | NA | NA |
| Hgb-7-42 | VIDAL VALLEY HIGHLANDS | NA | NA |
| Hgb-7-43 | CHEMEHUEVI VALLEY HIGHLANDS | 520 | yes |
| Hgb-7-44 | NEEDLES VALLEY HIGHLANDS | 665 | yes |
| Hgb-7-45 | PIUTE VALLEY HIGHLANDS | NA | NA |
| Hgb-7-46 | CANEBRAKE VALLEY HIGHLANDS | 842 | yes |
| Hgb-7-47 | JACUMBA VALLEY HIGHLANDS | 807 | yes |
| Hgb-7-48 | HELENDALE FAULT VALLEY HIGHLANDS | 666 | yes |
| Hgb-7-49 | PIPES CANYON FAULT VALLEY HIGHLANDS | 900 | yes |
| Hgb-7-5 | CHUCKWALLA VALLEY HIGHLANDS | 1048 | yes |
| Hgb-7-50 | IRON RIDGE AREA HIGHLANDS | NA | NA |
| Hgb-7-51 | LOST HORSE VALLEY HIGHLANDS | 772 | yes |
| Hgb-7-52 | PLEASANT VALLEY HIGHLANDS | NA | NA |
| Hgb-7-53 | HEXIE MOUNTAIN AREA HIGHLANDS | NA | NA |
| Hgb-7-54 | BUCK RIDGE FAULT VALLEY HIGHLANDS | NA | NA |
| Hgb-7-55 | COLLINS VALLEY HIGHLANDS | 1075 | yes |
| Hgb-7-56 | YAQUI WELL AREA HIGHLANDS | 915 | no |
| Hgb-7-59 | MASON VALLEY HIGHLANDS | 638 | yes |
| Hgb-7-6 | PINTO VALLEY HIGHLANDS | 650 | yes |
| Hgb-7-61 | DAVIES VALLEY HIGHLANDS | NA | NA |
| Hgb-7-62 | JOSHUA TREE HIGHLANDS | 878 | no |
| Hgb-7-63 | VANDEVENTER FLAT HIGHLANDS | 1154 | yes |
| Hgb-7-7 | CADIZ VALLEY HIGHLANDS | 1015 | yes |
| Hgb-7-8 | BRISTOL VALLEY HIGHLANDS | 719 | no |
| Hgb-7-9 | DALE VALLEY HIGHLANDS | 845 | yes |
| Hgb-8-1 | COASTAL PLAIN OF ORANGE COUNTY HIGHLANDS | 959 | no |
| Hgb-8-2 | UPPER SANTA ANA VALLEY HIGHLANDS | 1246 | yes |
| Hgb-8-4 | ELSINORE HIGHLANDS | 1643 | no |
| Hgb-8-5 | SAN JACINTO HIGHLANDS | 1867 | yes |
| Hgb-8-6 | HEMET LAKE VALLEY HIGHLANDS | 1082 | yes |
| Hgb-8-7 | BIG MEADOWS VALLEY HIGHLANDS | 513 | no |
| Hgb-8-8 | SEVEN OAKS VALLEY HIGHLANDS | 874 | yes |
| Hgb-8-9 | BEAR VALLEY HIGHLANDS | 736 | no |
| Hgb-9-1 | SAN JUAN VALLEY HIGHLANDS | 1827 | yes |
| Hgb-9-10 | SAN PASQUAL VALLEY HIGHLANDS | 2036 | yes |
| Hgb-9-11 | SANTA MARIA VALLEY HIGHLANDS | 1734 | yes |
| Hgb-9-12 | SAN DIEGUITO CREEK HIGHLANDS | 1715 | yes |
| Hgb-9-13 | POWAY VALLEY HIGHLANDS | 1917 | yes |
| Hgb-9-14 | MISSION VALLEY HIGHLANDS | 1879 | yes |
| Hgb-9-15 | SAN DIEGO RIVER VALLEY HIGHLANDS | 1707 | yes |
| Hgb-9-16 | EL CAJON VALLEY HIGHLANDS | 1948 | yes |
| Hgb-9-17 | SWEETWATER VALLEY HIGHLANDS | 1858 | yes |
| Hgb-9-18 | OTAY VALLEY HIGHLANDS | 1809 | yes |
| Hgb-9-19 | TIA JUANA HIGHLANDS | 395 | no |
| Hgb-9-2 | SAN MATEO VALLEY HIGHLANDS | 1822 | yes |
| Hgb-9-22 | BATIQUITOS LAGOON VALLEY HIGHLANDS | 930 | no |
| Hgb-9-23 | SAN ELIJO VALLEY HIGHLANDS | 1945 | yes |
| Hgb-9-24 | PAMO VALLEY HIGHLANDS | 1530 | yes |
| Hgb-9-25 | RANCHITA TOWN AREA HIGHLANDS | 1241 | yes |
| Hgb-9-27 | COTTONWOOD VALLEY HIGHLANDS | 1198 | no |
| Hgb-9-28 | CAMPO VALLEY HIGHLANDS | 1370 | no |
| Hgb-9-29 | POTRERO VALLEY HIGHLANDS | 1384 | yes |
| Hgb-9-3 | SAN ONOFRE VALLEY HIGHLANDS | 525 | no |
| Hgb-9-32 | SAN MARCOS AREA HIGHLANDS | 1907 | no |
| Hgb-9-4 | SANTA MARGARITA VALLEY HIGHLANDS | 1477 | yes |
| Hgb-9-5 | TEMECULA VALLEY HIGHLANDS | 1673 | yes |
| Hgb-9-6 | CAHUILLA VALLEY HIGHLANDS | 1482 | yes |
| Hgb-9-7 | SAN LUIS REY VALLEY HIGHLANDS | 1591 | yes |
| Hgb-9-8 | WARNER VALLEY HIGHLANDS | 1215 | yes |
| Hgb-9-9 | ESCONDIDO VALLEY HIGHLANDS | 1643 | no |
| Hp-1.1 | Northern Coast Ranges - North Coast | 422 | yes |
| Hp-1.2 | Northern Coast Ranges - Central Coast | 549 | yes |
| Hp-1.3 | Northern Coast Ranges - Southern Coast | 532 | yes |
| Hp-1.4 | Northern Coast Ranges - Point Reyes | 627 | yes |
| Hp-10.1 | Desert - Eastern | 555 | yes |
| Hp-10.2 | Desert - Southern | 1031 | no |
| Hp-11 | Southern Cal Islands (Group) | 140 | no |
| Hp-2 | Klamath Mountains | 673 | yes |
| Hp-3 | Modoc Plateau and Cascades | 1005 | yes |
| Hp-4.1 | Sierra Nevada - Tahoe | 1302 | yes |
| Hp-4.2 | Sierra Nevada - Alpine County | 817 | yes |
| Hp-5 | Basin and Range | 593 | yes |
| Hp-6 | Northern and Southern Coast Ranges - San Francisco Bay and Delta | 672 | yes |
| Hp-7.1 | Southern Coast Ranges - Santa Cruz / Half Moon Bay | 930 | no |
| Hp-7.2 | Southern Coast Ranges - Big Sur/Carmel | 1313 | yes |
| Hp-7.3 | Southern Coast Ranges - San Luis Obispo | 736 | no |
| Hp-8.1 | Transverse and Selected Peninsular Ranges - Santa Barbara | 1380 | yes |
| Hp-8.2 | Transverse and Selected Peninsular Ranges - Malibu | 1347 | yes |
| Hp-8.3 | Transverse and Selected Peninsular Ranges - Palos Verdes | NA | NA |
| Hp-9.1 | San Diego Drainages - North San Diego Coastal | 816 | no |
| Hp-9.2 | San Diego Drainages - South San Diego Coastal | 1408 | yes |
| Hp-9.3 | San Diego Drainages - South San Diego | 1558 | yes |

The well record information is from the Department of Water Resources Online System for Well Completion Reports.

The small water system location data is from the Division of Drinking Water.

Asbestos, fecal coliform, and total coliform are not assessed for the aquifer risk map although these constituents have an MCL.

The comparison concentration values for chemicals without an MCL are as follows: Hexavalent Chromium – 10 micrograms per liter (µG/L); Copper – 1.3 milligrams per liter (MG/L); Lead – 15 µG/L; N-Nitrosodimethylamine (NDMA) – 0.1 µG/L.

See page 5 of the Domestic Well Needs Assessment White Paper. The MCL index consists of the finding divided by the MCL, with a special consideration for non-detect results with a reporting limit above the MCL.

This project uses Groundwater Units as areas of analysis. Groundwater Units consist of groundwater basins as defined by DWR Bulletin 118, and the connecting upland areas associated with each of these basins as delineated by the USGS. Use of Groundwater Units results in coverage of the entire state. Averaging of well depths and groundwater quality within a Groundwater Unit was considered reasonable based on the assumed relative consistency of hydrogeologic conditions within each Unit.

Domestic Bottom = average of section maximum domestic well depths (from OSWCR) plus 3 standard deviations of section maximum well depths for each groundwater unit.

Domestic Top = average of section minimum domestic well depths (from OSWCR) minus 3 standard deviations of section minimum well depths for groundwater unit.

Public Bottom = average of section maximum public well depths (from OSWCR) plus 3 standard deviations of section maximum well depths for groundwater units.

To calculate the 20-year average, water quality results with sample collection dates between January 1, 2001 and January 1, 2021 were used.

To calculate results within the last five years, water quality results with sample collection dates between January 1, 2016 and January 1, 2021 were used.

For this map, on the “source” and “neighbor” sections described in the Domestic Well Needs Assessment White Paper are used. The White Paper also describes the calculations for “groundwater unit” sections, which are not included in this map.